GUIDANCE  MATERIAL

ON

SACCAN V2 FANS 1/A

OPERATIONAL STATUS

in

CANARIAS  airspace

TABLE OF CONTENTS

PREFACE………………………………………………….………………………………….…	 2

LIST OF ACRONYMS………………………………………….……………………….………	 3

PART 1 – SACCAN V2 FANS 1/A OPERATIONAL STATUS       ………………..…...	 5

  - SACCAN V2 General Description and Current Status…………………...…………	 5

  - SACCAN V2 Background…………………………………………………………..	 7

  - CANARIAS Current Limitations and Need for Improvement……………….………….	 7

  - SACCAN V2 Operational Concepts and Functions…………………………………	 7

  - SACCAN V2 General Objectives……………………………………………………..	10

  - SACCAN V2 Expected Benefits…………………………...……………………….	10

  - SACCAN V2 System Architecture and  Blocks Description….…………………….	11

  - Conduct of SACCAN V2 FANS 1/A Operations    …. … .. .. … .. .. .. .…............	12

  - Area of Applicability……………………………………….……………………………	13

  - Contacts………………………………………………….………………………………	13

PART 2 – SYSTEM OPERATION – MANAGING ADS AND CPDLC…………….…………	15

2.1	Log-on/Connection………………………………………………………………………	15

2.2	Establishing and Terminating an ADS Connection…………………...…………………	16

2.3	ADS Emergency Mode Operation……………………...………………………………..	17

2.4	Surveillance Safety Considerations……………………...………………………………	17

2.5	Establishing, Executing, and Terminating a CPDLC Connection………….…………..	17

2.6	Communications Safety Considerations……………………………...………………….	20

PART 3 – RIGHTS AND RESPONSIBILITIES……………………………..………………….	21

3.1	ATS Provider Rights and Responsibilities………………………………..…….………..	21

3.2	Operator Rights and Responsibilities…………………………………………….………	21

3.3	Communications Service Provider Responsibilities…………………..…………………	22

PART 4 – OPERATIONAL PROCEDURES………………………. …..……..……	23

4.1	Flight Planning Procedures…………………………………………………..…………..	23

4.2	Air Traffic Controller Procedures……………………………………………………….	24

4.3	Flight Crew Procedures………………………………………………………………….	24

APPENDIX 1 – SACCAN V2 System Architecture and Blocks Description .………………APP1-1

APPENDIX 2 – Downlink and Uplink Messages ……..………..………….……....………...APP2-1

APPENDIX 3 – Dialogues Examples ………….………………………...…...……..……....APP3-1

APPENDIX 4 – FANS-1/A CPDLC Message Set and Intent …………...…...……..……....APP4-1

FIGURES:

Figure 1 – SACCAN V2 FANS 1/A System Description…………………………..……………..	 6

Figure 2 – SACCAN V2 Logical Blocks……………………………….……………………….APP1-1

PREFACE

This Document details the SACCAN V2 system, and the procedures and requirements applicable to the Operational Status of SACCAN FANS 1/A in Canarias airspace. Information relating to the CANARIAS Operational Status  is contained in a State AIC which should be read in conjunction with this Document.

In this Operational status CPDLC messages will be used to satisfy real operational needs,  controller instructions via CPDLC and any other messages that may affect safety will not require to be confirmed via voice communications, and will have the same executive level as radio-transmitted instructions.

To assist with the editing of this Document and to ensure the currency and accuracy of future editions, comments/suggestions for possible amendments should be sent to the editor at the following EMAIL address or facsimile number:

-	email: FTravieso@aena.es

-	fax: + 34 928577057 (Attn: Félix Travieso)

LIST OF ACRONYMS

| ACC | Area Control Centre |
| --- | --- |
| ACID | Aircraft Identification |
| ADS ADS-C AENA | Automatic Dependent Surveillance ADS Contract Spanish Airports and Air Navigation |
| AFN | Air Traffic Services Facilities Notification |
| AIP AIC | Aeronautical Information Publication Aeronautical Information Circular |
| AMP | Aircraft Messages Processing |
| ARINC | Aeronautical Radio INC. |
| ATC | Air Traffic Control |
| ATM | Air Traffic Management |
| ATS | Air Traffic Services |
| ATSU | Air Traffic Services Unit |
| CAA | Civil Aviation Authority |
| CNS COM | Communications, Navigation and Surveillance Communications Processing |
| CPDLC | Controller Pilot Data Link Communications |
| DL DLT ECA | Data Link Data Link Terminal ADS-SSR Control Station |
| EMG | Emergency Message |
| FANS | Future Air Navigation Systems |
| FANS 1 | Boeing implementation of FANS |
| FANS A | Airbus implementation of FANS |
| FANS 1/A | Boeing & Airbus implementations of FANS |
| FDP | Flight Plan Data Processor |
| FDPS | Flight Data Processing System |
| FIR | Flight Information Region |
| FMC | Flight Management Computer |
| FMS | Flight Management System |
| GPS | Global Positioning System |
| HF | High Frequency |
| ICAO | International Civil Aviation Organization |
| ISPACG | Informal South Pacific ATS Coordinating Group |
| LAN LRNS | Local Area Network Long Range Navigation System |
| MET MSAW MTCA | Meteorological Minimum Safe Altitude Warning Medium Term Conflict Alert |
| NIM | Navigation Integrity Monitoring |
| NOTAM | Notice to Airmen |
| POS PSR | ICAO Position Report Message Primary Surveillance Radar |
| RNP RRU SACCAN SACTA | Required Navigation Performance Radar Data Reception Unit CANARIAS ADS/CPDLC System Air Traffic Control Automated System |
| SARPS SATMA SDP | Standards and Recommended Practices South Atlantic Monitoring Agency Surveillance Data Processing |
| SITA SSR STCA SVT TDT UTC | Societe Internationale de Telecomunications Aeronautiques Secondary Surveillance Radar Short Term Conflict Alert Supervision Terminal Traffic Display Terminal Universal Time Coordinated |
| VHF | Very High Frequency |
| WP | Waypoint Position |
| WPR | Waypoint Position Reporting |

# – SACCAN V2 FANS 1/A OPERATIONAL STATUS

## 1.1	SACCAN V2 General Description and Current Status

1.1.1		SACCAN V2 is a ground ADS/CPDLC system of Aena currently sited at the operations room of CANARIAS ACC. The main purpose of SACCAN V2, after proper operational evaluation and validation, is to provide air traffic control services to FANS 1/A aircraft operating in the CANARIAS airspace. ICAO compliant aircraft (CNS/ATM package 1) will be accommodated later when they start operating.

    - FANS 1/A aircraft equipage normally includes VHF&SATCOM Data Link, ADS, CPDLC, FMS, and a Navigation Package.

    - The FANS-1/A Navigation Package normally consists of three Inertial Reference Systems providing position and velocity information to two Long Range Navigation Systems (LRNS) contained in individual Flight Management Systems each of which also references its own Global Positioning System (GPS) receiver and VOR/DME/ILS signals.  These sensor inputs are resolved into a single aircraft position solution within each LRNS. This navigation package is capable of being approved for RNP 4. The actual navigation performance is constantly monitored; if it exceeds the required navigation performance (RNP), the flight crew is alerted.

    - The technical specification for CPDLC is provided in RTCA DO-219.  The technical specification for the bit to character conversion and the ATS Facilities Notification (AFN) application is provided in AEEC 622-2.  Deviations from the aforementioned standards in specific airframe implementations are provided in the manufacturers interoperability documents, the Boeing Air Traffic Services System Requirements and Objectives document (ATS/SR&O) and the Airbus AIM FANS System Objectives and Requirements document (FANS-A SO&R).

    - The technical specification for ADS is provided in AEEC 745-2 and RTCA DO-212. The technical specification for the bit to character conversion and the ATS Facilities Notification (AFN) application is provided in AEEC 622-2.  Deviations from the aforementioned standards in specific airframe implementations are provided in the manufacturers interoperability documents, the Boeing ATS/SR&O and the Airbus FANS-A SO&R.

    - FANS 1/A equipped aircraft use the SITA and ARINC networks and can communicate with SACCAN V2 by means of the Aeronautical Mobile Satellite Service (AMSS) provided by INMARSAT, or by VHF when within the range of any of the multiple SITA or ARINC VHF data link stations, like the two of SITA located in the Canary Islands.

    - A most outstanding characteristic of SACCAN V2 is its capability to establish, change and cancel ADS contracts automatically. This is done in order to minimize the air-traffic controller workload required to operate ADS.

    - All SACCAN V2 technical acceptance tests have carried out successfully and therefore next phase was operational evaluation with both real and simulated traffic, ending up in the current Operational Status.

    - Inter-operability tests have been carried out successfully with both the FANS 1 Boeing  747&777 simulator in Seattle and with the FANS A  Airbus A340 simulator in Toulouse.

    - An ADS/CPDLC simulator capable of simulating data from up to forty aircraft has been developed and was used in the operational evaluation of SACCAN V2 with simulated traffic.

GOONHILLY

AUSSAGUEL

ARINC

CANARIAS

FIR

SITA

INMARSAT

CANARIAS ACC

FANS1/A

= VHF DATA LINK

FIGURE 1:SACCAN V2 FANS 1/A SYSTEM DESCRIPTION

## 1.2	SACCAN V2 Background

1.2.1 	The SACCAN V2 system installed in the control/operations room of CANARIAS ACC is in fact a reduced replica of a larger system called ECA, used by Aena (Spanish Airports and Air Navigation) for operations, experimentation, evaluation and validation of ADS, CPDLC, and other advanced data link applications and functions. Aena made use of ECA during February and March 1998 to participate in ADS Europe trials using the Aeronautical Telecommunication Network (ATN).

1.2.2	SACCAN V2 as well as ECA incorporates the philosophy, tracking algorithms, and functions of the  ADS-SSR Integration Study carried out by Aena in 1994. The experience gained during the PRODAT/PROSAT programme of the European Space Agency (ESA) in which Spain participated was also taken on-board during system specification.

## 1.3	CANARIAS Current Limitations and Need for Improvement

### 1.3.1	Canarias has an airspace only partially covered by radar where a lot of  over-flights and departing/arriving aircraft from/to the islands operate.

1.3.2	There are large parts of airspace out of coverage of the ground-based systems sited in the islands, where no radar surveillance is available, and where HF is the only means for voice position reporting and other controller-pilot communications; As a consequence large separations and complex operational procedures, such as the Mach number technique, has to be applied in many cases.

1.3.3	Of course, in the area covered by radar more efficient separations are applied to arriving and departing aircraft, although large longitudinal separation must anyhow be established before leaving radar coverage outbound.

1.3.4	Is evident that there is an operational need, to improve the current situation that does not allow for a flexible and efficient use of the airspace, and get rid of complex operational procedures which generate a lot of extra workload to both air-traffic controllers and pilots.

## 1.4	SACCAN V2 Operational Concepts and Functions

The following main new concepts and functions are available within SACCAN V2:

- ADS & ADS-SSR tracking;

- STCA and MSAW based on ADS & ADS-SSR tracking;

- Navigation Integrity Monitoring (NIM);

- Detection  of Incorrect Waypoint Insertion;

- Conformance monitoring & Automatic update of flight plan by the tracker;

- Automatic activation of ADS contracts;

- Automatic cancellation of ADS contracts;

- Central Management  of ADS contracts;

- Manual management and visualization of ADS contracts;

- Generation and display of flight plan tracks;

- Controller-Pilot Data Link Communications (CPDLC); and

- GPS availability visualization.

1.4.1	ADS &ADS-SSR tracking

1.4.1.1	An adaptable tracking algorithm which integrates into one common track ADS-C data and SSR data from up to eight radars is available within SACCAN V2. This algorithm can also work with only ADS-C or radar only data.

1.4.1.2	The most outstanding feature of this adaptable tracking algorithm is its capability to adapt, in real time, the reporting rate of ADS-C periodic contracts to the tracking accuracy and redundancy required. When ADS data is not required the reporting rate is set to a minimum value, just to keep the periodic contract alive and running with a minimum cost. When ADS becomes necessary, because the number of radars is not enough to meet the redundancy requirement or the radar data is not accurate enough to meet the accuracy requirement, the reporting rate is increased accordingly. This can be very cost-effective since the reporting rate can be kept to the minimum necessary to fulfil the requirements.

1.4.1.3	The accuracy and redundancy requirements can be specified by the SACCAN V2 system operator either for each of the boxes of an airspace requirements mosaic, for one particular aircraft, or for one or various aircraft group/s. The accuracy has to be specified in meters and the redundancy must reflect the number of surveillance sources required to track the aircraft.

1.4.1.4	A non-adaptable ADS-SSR tracking algorithm is also available within SACCAN V2.

1.4.1.5	Both the adaptable and non-adaptable tracking algorithms have the capability to estimate the uncertainty/accuracy of the aircraft position being displayed to the controller. This is a most important feature in order to assess the minima separation that could be applied each moment to aircraft. In the case of ADS tracking, position uncertainty, which can be represented by a circle, is a function of the figure of merit, time elapsed from the time stamp of last ADS report received, and aircraft speed. Is evident that the maximum size that the circle can take, assuming no losses, is also a function of the ADS periodic contract-reporting rate.

1.4.1.6	The main purpose of the tracking function is to improve the accuracy and availability of the positional data (ADS and/or SSR data) being received from aircraft. It minimizes the noise of the position received by means of track smoothing, compensates for positional data losses by using position extrapolations instead, and calculates the speed of aircraft when not provided.

1.4.2	STCA and MSAW based on ADS & ADS-SSR tracking

1.4.2.1	Short Term Conflict Alert and Minimum Safe Altitude Warning based on ADS tracking and/or ADS-SSR tracking are also available. The STCA and MSAW modules themselves are capable of adapting in real time the reporting rate and content of ADS-C periodic contracts to its own requirements which will be more demanding as aircraft come more and more closer to each other or to an specified area. This is an enhancement of the classic STCA and MSAW based only on radar since the number of false alerts will be minimized because of the availability, on request, of accurate attitude data as provided by the earth reference group (true track, ground speed and vertical rate) of ADS.

1.4.3	Navigation Integrity Monitoring (NIM)

1.4.3.1	A function is available to crosscheck ADS positional data [derived from the aircraft navigation system (GPS, VOR, etc.)] with SSR positional data of the same aircraft. This enables the ATC to detect errors of the navigation system being used by the aircraft and therefore control its integrity. An alert is displayed when ADS and SSR positional data from the same aircraft do not match each other within a margin that can be set by the system operator. The operator can also set the time between crosschecks.

1.4.4	Detection of Incorrect Waypoint Insertion

1.4.4.1	The availability of the next two waypoints (predicted route group), as provided by ADS-C, permits the ground system (ATC) to crosscheck this data with the flight plan route data stored in the ground Flight Plan Data Processor (FDP) in order to detect possible incorrect waypoint data insertion before a dangerous situation may arise. The ground system operator can set the time between crosschecks.

1.4.5	Conformance monitoring & Automatic update of flight plan by the tracker

1.4.5.1 When differences (lateral deviations) between tracking data (ADS, ADS-SSR, or SSR) and the route data of the flight plan data stored in the flight plan data processing system exceed a pre-defined tolerance limit (currently set to 5NM for lateral deviations) an out-of-conformance indication (a change in the color of the track) is displayed to the controller

1.4.5.2	Based on tracking data (ADS, ADS-SSR or SSR) the SACCAN V2 system automatically updates the passing times and estimates over the route fixes of the flight plan data processing system.

1.4.6	Automatic activation of ADS contracts

1.4.6.1	Immediately after an aircraft logs on to SACCAN V2, an initial ADS-C periodic contract with a low reporting rate is automatically established with that aircraft by the ground system. Once a track (ADS or ADS-SSR) has been created, the tracker takes control, adapting if required the reporting rate of the initial contract to its own needs (requirements) as explained before

1.4.7	Automatic cancellation of ADS contracts

1.4.7.1	An adaptable parameter of time after an aircraft has left or landed at Canarias FIR, cancellation of all ADS contracts established with that aircraft is done automatically by SACCAN V2. Manually cancellation can also be done by controller at any time.

1.4.8	 Central Management  of ADS contracts

1.4.8.1	This function implemented in SACCAN V2 centralizes all system needs for ADS-C data. It takes from all system modules the requests for ADS data  ( ADS & ADS-SSR Tracker, STCA, MSAW, NIM, FDP, Operator, etc.), and based on that builds and establishes only one periodic and one event contract per aircraft to satisfy all needs. Contracts are changed when necessary by the system.

1.4.8.2	As explained above ADS contracts management (activation, cancellation, change, etc.) is handle automatically by the system itself in order to minimize the air traffic controller workload required for operating ADS. Nevertheless a manual function is available for the operator to make his own ADS data requests, supervise, change and cancel contracts in place.

### Manual management and visualization of ADS contracts

1.4.9.1	This manual function allows the controller to establish, modify, cancel and visualize: periodic, event, and demand contracts, as supported by FANS 1/A equipped aircraft.

1.4.10	Generation and display of flight plan tracks

1.4.10.1 When this function is enabled, synthetic tracks based on flight plan data stored in the flight plan data processor are created, maintained, and displayed to the controller. This is only done for aircraft for which no real tracks (ADS, ADS-SSR, SSR, or PR) exist. Of course, any manual update of estimates done by the controller as consequence of a voice/CPDLC position report received, is immediately reflected on the track. Synthetic tracks are displayed using a specific position symbol and colour so as to differentiate them from real tracks. The availability of synthetic tracks will eliminate the workload required by the controller to built an image of aircraft positions in his mind based only on flight plan data and voice position reports.

1.4.11	Controller Pilot Data Link Communications (CPDLC)

1.4.11.1 Both SATCOM and VHF data links within its respective areas of coverage can support controller-pilot data link messages exchange. A mouse and a keyboard permit air-traffic controllers to easily and quickly compose CPDLC messages using canned-coded messages. A free text editing function is also available.

1.4.12	GPS availability visualization

1.4.12.1This function calculates and displays the theoretical number of expected GPS satellites in sight at any position of the CANARIAS FIR selected by means of mouse. Its purpose is to give air traffic controllers an idea of what would be the availability and continuity of the ADS function, when based on GPS, in a particular area of airspace before aircraft come in. In future is expected that this function will be connected to EGNOS to provide real time data.

## 1.5	SACCAN V2 General Objectives

1.5.1	The SACCAN V2 general objectives were:

- Familiarize air traffic controllers and technicians of CANARIAS ACC with the ADS and CPDLC applications;

- Evaluate and validate the new functions and operational procedures that will be required for efficient ADS/CPDLC use;

- Asses, based on real data, the safety, efficiency and economical benefits to be derived from the application of ADS and CPDLC in the Canary Islands

As a result of above, in the current Operational Satus ADS and CPDLC services to FANS 1/A and ICAO compliant aircraft operating in the CANARIAS FIR/UIR are being provided.

1.6	SACCAN V2 Expected Benefits

1.6.1	The implementation of FANS1/A services in the Canary Islands FIR is expected to bring a series of benefits deriving from a more accurate navigation using GPS/RNAV, a more efficient communications system (CPDLC), and ADS.

1.6.2	The following improvements are expected:

- More direct flight paths;

- More optimal climb and descend profiles;

- Increased access to cruise altitudes closer to optimal;

- Reduced air-traffic controllers and pilots workload; and

- Increased level of safety.

1.6.3	For aircraft operators, the above improvements will generate savings in fuel and other operating costs, such as crew costs. For passengers, will reduce travel time.

1.6.4	The above improvements will be based on the following:

- A much more accurate navigation using GPS that will permit to establish a much more efficient route structure characterized by the extensive use of direct routes and more closely spaced airways. The elimination of two-opposite-directions airways will allow optimum climbing and descending profiles which will not be staggered since aircraft will not be restricted by traffic in the opposite direction, and will also increase safety and reduce pilot and controller workload;

- A reduction in controllers workload per aircraft which will be the result of the elimination of voice position reporting and complex operational procedures, a less problematic route structure, implementation of new automated functions, and the extensive use of CPDLC. This reduction in workload will make possible for the controller to increasingly authorize more direct trajectories and attend a larger number of aircraft when so required;

- Improved surveillance based on ADS and on its integration with radar, which will make possible to the controller apply more reduced separation minima and smaller longitudinal separation, since greater levels of accuracy, availability, integrity and continuity of serviced will be ensured. The reduction in longitudinal separation will require in most cases an equivalent implementation in the other FIRs of the Region; and

- Detection of incorrect waypoint insertion, navigation integrity monitoring, and STCA/MSAW based on ADS.

1.6.5	Although the above benefits will not be very significant until a high proportion of aircraft will be equipped, those equipped will enjoy since the very beginning a reduction in pilot communications workload, and a higher level of safety. Some direct routing approvals will be also possible from the beginning since its negotiation (by CPDLC) and surveillance (by ADS) will not impose a significant workload to the controller.

1.6.6	The early safety benefits will come from the “detection of incorrect waypoint insertion”, the “conformance monitoring”, and the “navigation integrity monitoring” functions of the SACCAN V2 system. CPDLC will also contribute to these benefits by minimizing current voice communications misunderstandings and language difficulties.

1.6.7	A way to maximize the early benefits might be traffic segregation. Specific routes and/or levels could be dedicated to  traffic properly equipped.

## 1.7	SACCAN V2 System Architecture and Blocks Description

1.7.1	SACCAN V2 system architecture and mayor logical blocks description can be found in Appendix 1

1.8	Conduct of the Second Phase of SACCAN V2 FANS 1/A Pre-Operational Evaluation Trials and Operational Status

    - The Spanish Air Navigation Service Provider, Aena, was ready to start the Pre-Operational phase of the SACCAN V2 FANS 1/A ADS&CPDLC with the interested aircraft operators in the Canarias FIR/UIR. First phase, in which Lufthansa, LanChile, TAM, and Air France were participating, started August 15th 2002 until November 9th, 2003. The Second Phase started November 10th, 2003 until December 18th, 2006.

1.8.2	That Pre-Operational phase started on July 5th, 2007 and lasted until establishment of the current operational status. ADS-C and CPDLC connection are available H24.  Changeovers or altered circumstances will be published by NOTAM.

    - The SACCAN V2 FANS 1/A Pre-Operational phase was monitored and coordinated by the already existing South Atlantic Monitoring Agency (SATMA) of Spain, as happens now in current Operational Status.

1.8.4	All airline operators with FANS 1/A equipped aircraft interested in operate with SACCAN v2 FANS 1/A in Operational Status are kindly requested to provide to SATMA the following information:

- operator name;

- operator contact person;

- aircraft type(s) and associated registration(s);

- whether the option of updating the FMC time using the GPS time has been installed for the particular aircraft involved;

- whether the option of participation is for “ADS only” or also for CPDLC but subject to CPDLC trained crew on board;

- anticipated start date of participation; and

- indication whether the approval(s) referred above in 1.8.3 cover(s) both ADS and CPDLC, or ADS only.

Additionally, for aircraft operating in CPDLC, the following was required to be provided:

- assurance that  operational approval has been obtained in accordance with FAA AC 120-70 and associated Operational Approval Information Package, or, if it exists, equivalent material;

1.8.5	A registration form used to file the required data to operate in the Operational Status is attached in Appendix 5 of the present document. Registration forms and any other required documentation should be submitted to the following SATMA address:

Agencia de Monitorización SATMA

Centro de Control de Tránsito Aéreo de Canarias

Apartado 69

E35219 Telde (Las Palmas de Gran Canaria) – ESPAÑA

Phone: 	+ 34-928577057

Fax:	+ 34-928577052

E-mail:	:   satma@aena.es

- A copy should also be sent to the following Aena headquarters address:

Departamento de Armonización Técnico Operativa

Dirección de Tránsito Aéreo

Aena

C/ Juan Ignacio Luca de Tena, 14 – 28027 Madrid

ESPAÑA

Phone: 	+ 34-913213297

Fax:	+ 34-913213181

- E-mail:	 rvega@aena.es

1.8.6	Any other request or documentation should be addressed to SATMA:

Agencia de Monitorización SATMA

Centro de Control de Tránsito Aéreo de Canarias

Apartado 69

E35219 Telde (Las Palmas de Gran Canaria) – ESPAÑA

Phone: 	+ 34-928577057

Fax:	+ 34-928577052

E-mail:	:   satma@aena.es

- A copy should also be sent to the following Aena headquarters address:

Departamento de Armonización Técnico Operativa

Dirección de Tránsito Aéreo

Aena

C/ Juan Ignacio Luca de Tena, 14 – 28027 Madrid

ESPAÑA

Phone: 	+ 34-913213297

Fax:	+ 34-913213181

- E-mail:	 rvega@aena.es

1.8.7	To avoid logons being rejected and to ensure FANS 1/A downlinks are properly routed, each operating airline must co-ordinate with its Data Link Service Provider (or providers if applicable) to initiate FANS 1/A ground system configuration for its aircraft.

1.8.8	Operation with CPDLC is subject to  CPDLC trained crews on board. It is the responsibility of the Operator to ensure that only trained crews manage CPDLC.

1.8.9	For a particular flight of an airline that has complied  with the specified requisites for CPDLC, operating with CPDLC can happen just before entering CANARIAS FIR/UIR by means of the appropriate CPDLC message. For flights southbound, Canarias OCE Sector will assure that they are connected ADS/CPDLC entering CANARIAS FIR/UIR, and will advise previous collateral Sectors.

1.9	Area of Applicability

1.9.1	Operational procedures of SACCAN V2 FANS 1/A will take place in the Canarias FIR/UIR. However, operational and technical structure in Canarias Operational Room will restrict ADS-CPDLC operations to the Oceanic sector (OCE).

1.10	Contacts

1.10.1	Aena-Canarias operates SATMA. Contact points for the SACCAN V2  FANS 1/A Operational Status within Aena-Canarias are as follows:

Canarias Coordinator - 		Name: Felix Travieso

Canarias ADS-C/CPDLC Coordinator, Chief of Support to Operations

Canarias ACC

Phone : + 34 928577057

Fax : + 34 928577052

Email : FTravieso@aena.es

Canarias ACC Operations -		Name: Miguel Torrens

Head of Division of the Regional ATS Services

Canarias ACC

Phone : + 34 928577050

Fax : + 34 928577052

Email : mtorrens@aena.es

1.10.2	Contact point for the SACCAN V2  FANS 1/A Operational Status within Aena-Headquarters is as follows:

Headquarters Coordinator - 	Name: Ramón Vega

Jefe Departamento de

Armonización Técnico Operativa

Aena - Madrid

Phone: 	+ 34-913213297

Fax:	+ 34-91321381

E-mail:	:    rvega@aena.es

1.10.3	Operators using ARINC as their Data Link Service provider, contact:

Name: Terry Anton

Phone:	410 266-4027

Fax:	410 573-3515

Email: txa@arinc.com

1.10.4	Operators using SITA as their Data Link Service provider, contact:

Name: Eduard Blasi

Phone : + 33 1 4641 2473

Fax : + 33 1 4641 1978

Email : eduard.blasi@sita.int

# – SYSTEM OPERATION - MANAGING ADS AND CPDLC

2.1	Log-on/Connection

### Aircraft operating with the SACCAN V2 FANS 1/A are kindly requested to manually log-on the Canary Islands SACCAN V2 system by sending an ATS Facilities Notification (AFN) Contact message, containing the 4 character ICAO code of the CANARIAS  ATS unit “GCCC” between 15 and 30 minutes prior to entering the CANARIAS FIR/UIR.

2.1.2	If a log-on attempt is not successful, wait at least 5 minutes before making a second attempt

2.1.3	For flights departing from airports adjacent to, or within CANARIAS FIR/UIR, the pilot should log-on prior to departure.

    - SACCAN V2 will accept the ATS Facilities Notification (AFN) Contact message from the aircraft and generate an AFN Acknowledgement.  The AFN Acknowledgement will indicate that both ADS and CPDLC applications are supported.

    - The AFN log-on is a prerequisite to any CPDLC or ADS connection.

2.1.6	The AFN  log-on serves the following purposes:

- To provide the ground system with the data link application context of the aircraft, namely: the data link applications supported on board (ADS, CPDLC), their version numbers, and the associated addresses ( in the FANS-1/A context, these are the ACARS addresses unique to each aircraft).

- To provide the ground system with information such as the flight identification (ATS personnel usually use flight identifications to identify flights) and aircraft registration number (the registration number is used by the FANS network as the destination address for data link messages to aircraft). This information will allow the correlation of the flight attempting to log-on with the corresponding flight data held by the ATS system (SACCAN V2). The aircraft logging-on will then be positively identified by the ATS system.

- To allow the SACCAN V2 system to establish both ADS and CPDLC connections where applicable; Some aircraft may decide to operate only in ADS-C, and therefore the only connection to be activated will be ADS-C.

    - The AFN log-on will be rejected if:

- the aircraft registration/flight identification pairing does not match the pairing contained in the flight plan;

- there is no aircraft registration included in the flight plan; or

c)    there is no flight plan in the ATS system (SACCAN V2) for that flight.

2.1.8	The flight identification  used for log-on must be exactly the same as the filed in the ATS flight plan.

## 2.2	Establishing and Terminating an ADS Connection

2.2.1		Immediately after log-on has been completed successfully SACCAN V2 will automatically activate the ADS connection and set an initial 15 minutes (adaptable value) periodic reporting rate contract with the aircraft.

2.2.2	During the aircraft transit through CANARIAS FIR/UIR airspace  different periodic contract reporting rates and data contents can be set. The SACCAN V2 automatic ADS contracts management  feature adapts the initial periodic contract to meet surveillance operational requirements (accuracy and redundancy operational parameters) previously set by the operator for the cells of an airspace mosaic.

2.2.3	Only one periodic contract will be established between the ATC unit and a particular aircraft at any one time. Whenever a new periodic contract is established, the previous periodic contract is replaced. The periodic contract will remain in effect until it is modified or cancelled.

    - Although several types of event contracts can be established, the most  commonly used will be:

a)   ADS Way Point Change Event;

- Altitude Range Event  that will be mostly triggered when the aircraft altitude is 200 feet either less than or greater than the maintaining cleared altitude in a RVSM environment airspace; and

- Lateral Deviation Change Event (LDCE) that will be mostly triggered when the aircraft’s actual position exceeds a lateral distance of 5 NM in a RNP-10 airways network, from the aircraft’s expected position on the active flight plan.

    - Demand contracts could also be established as “one-off” request from the ground system to provide just one ADS report containing specific data as defined in the request. A demand contract can be set by the ground system at any time.

    - The ADS MET Data contract (i.e. a contract for periodic reporting of the Meteorological Group data with a typical reporting period of 30 minutes) can also be requested.

    - In order to minimize the cost of data communications the use of high periodic  reporting rates (the highest possible is 64 seconds for FANS 1/A avionics) and in general the amount of ADS data exchanged will be kept to the minimum required.

    - FANS 1/A equipped aircraft can have up to four (or five, depending on the avionics) ADS connections established, each with a different ground facility. All ADS connections have equal status within the avionics.

    - ADS contracts and connection will be terminated during this phase by the ground system :

- Automatically  when the aircraft has crossed outbound the Canaries FIR /UIR boundary;

- Automatically when the aircraft’s flight plan has been cancelled or has finished; or

c)	Manually by the controller at any time considered appropriate (The aircraft has landed within the Canaries FIR, under radar coverage, etc.)

During the Operational status it is not expected to execute any transfer since no adjacent ATS unit FANS 1/A equipped is yet available. The termination of ADS contracts with an aircraft, whether performed automatically or manually, will be strictly monitored to avoid  the unnecessary exchange of ADS data .

## 2.3	ADS Emergency Mode Operation

    - When SACCAN V2 receives an emergency-mode ADS report (triggered by pilot action) , it will alert the Controller. If a periodic contract is active, the emergency reports will be transmitted at the existing periodic rate. Otherwise, the rate will default to 304 seconds for Boeing aircraft or 64 seconds for Airbus aircraft. Only the pilot can cancel the emergency mode.

## 2.4	Surveillance Safety Considerations

2.4.3		As for radar is defined in ICAO Doc. 4444, ADS monitoring has to be understood as the use of ADS for the purpose of providing aircraft with information and advice relative to significant deviations from nominal flight path, including deviations from the terms of their air traffic control clearances.

    - The application of ADS separations, foreseen for another operational phase, will require extensive evaluation to be done with simulated traffic, development of appropriate operational procedures, and safety analysis.

2.5	Establishing, Executing, and Terminating a CPDLC Connection

### Properly approved for CPDLC aircraft see 1.8.6  h) and 1.8.9  wanting to operate with CPDLC, once entered Canarias FANS1/A airspace (usually Oceanic Sector) and correctly LOGGED ON, could contact Canarias ACC by means of CPDLC directly.

    - When receiving the aircraft message the controller will manually trigger (a mouse click) the CPDLC initiation in the SACCAN V2 system and will answer via CPDLC.

    - If when receiving the aircraft request, CPDLC communications are not possible with that aircraft, the Controller will transmit via voice: “UNABLE TO PERFORM CPDLC”. In this case, only voice communications will take place until further notice.

    - If when receiving the aircraft request CPDLC communications are not possible, but they are expected to be available later before the aircraft leaves the Canarias airspace, he/she will answer via voice “ROGER, I WILL CALL YOU BACK FOR CPDLC”, or “EXPECT CPDLC CONECTION AT  TIME __ __”.

    - A CPDLC exchange can only occur after the AFN logon has been completed, the CPDLC initiated by means of a CONNECTION REQUEST message triggered manually by the controller and sent by the SACCAN V2 system to the aircraft, and a CONNECTION CONFIRM message received from the aircraft.

    - When so requested by ATC by means of the free text message “START CPDLC WAYPOINT POSITION REPORTING” pilots shall ensure that a CPDLC message will be issued whenever an ATC waypoint is passed over (Waypoint Change Events (WCE) ). ATC expects position reports based on downlink message  POSITION REPORT position report  (Message 48 of the DOWNLINK MESSAGE ELEMENT TABLE defined in RTCA DO-219 that also can be seen in Appendix 4). Pilots should not expect a controller response to these position reports. Free text message “STOP CPDLC WAYPOINT POSITION REPORTING” will be used by ATC to advice pilot to stop this reporting.

    - A set of CPDLC messages to be used during this Operational status is in Appendix 2. This set is made of a limited number of the messages contained in the “FANS-1/A CPDLC Message Set defined in RTCA DO-219, MOPS for ATC Communications Application”. For any message not included in this Appendix, CPDLC text transcription of relevant standard OACI R/T phraseology procedures will be applicable.

    - Appendix 4 contains a complete listing with the message intent (interpretation) for all       FANS-1/A CPDLC messages (RTCA DO-219) as was defined by the ADS Panel. This is most useful in order to have a clear understanding of the intent (meaning) of the CPDLC messages.

    - Operating with CPDLC does not release pilots from the obligation of establishing, maintaining, and monitoring HF or VHF voice communications.

    - Pilots operating with CPDLC must have available the  CPDLC Set contained in Appendix  2  of this Guidance Material Document. Only those messages held at Appendix 2 should be used for CPDLC. For messages out of Appendix 2 ICAO R/T voice phraseology procedures will continue to be used.

    - The CPDLC set in Appendix 2 is divided into two subsets. The first subset contains the “pilot messages, and the second subset the “controller messages”.

    - CPDLC messages exchanged during this Operational status will be only used to satisfy real “operational needs”, and not simply for the sake of testing and familiarisation. Therefore a pilot participating in a CPDLC session will never make any  request via  CPDLC unless he has a real need. For example, he will never send the CPDLC message “request climb to FL”,  unless he really wants to climb.

    - Pilots should not expect controller voice read-backs to pilot CPDLC requests, but direct CPDLC responses. If a response to a request is not received within a reasonable time  pilot should revert to R/T to repeat the request via voice.

NOTE: See Example 1 and Example 2 in Appendix 3

    - Controller instructions via CPDLC are be complied with. Pilots should send a reply data link message (i.e.,“WILCO”, or an instruction read-back) and  carry out the instruction.

    - At any moment during a CPDLC session pilot and controller will be free to use R/T voice instead of a particular CPDLC message of Appendix 2 if they consider it more appropriate at that moment, because of work overload, urgency, or any other reason. This does not imply that the CPDLC session has to be terminated unless a decision has been taken not to use CPDLC any longer during the flight.

    - The termination of a CPDLC session may be decided at any moment by either pilot or controller and will be communicated to each other by means of the following free text message:

“REQUEST CPDLC TERMINATION”

The response to this message should  be the message “ROGER”.

In the event of work overload or urgency, voice could be used instead.

    - In order to terminate CPDLC the controller will trigger the initiation of the CPDLC connection termination sequence which will cause SACCAN V2 to send an END SERVICE uplink message. In response to this message:

- The avionics will downlink a DISCONNECT message, and the avionics will consider the aircraft to be disconnected as soon as the DISCONNECT message is sent.

- The active CPDLC connection will be terminated by the activation of a non-active connection if existing.

    - The controller shall ensure that no open uplink CPDLC messages exist prior to the up-linking of an END SERVICE message.

    - Although it is an abnormal case, if the controller is aware that the sending of the END SERVICE message did not occur or has been unsuccessful, the pilot will be instructed by voice to terminate the connection.

    - If the CPDLC connection does not terminate automatically at the appropriate time (normally 5 minutes after leaving CANARIAS FIR/UIR), then the pilot shall manually disconnect.

2.6	Communications Safety Considerations

    - During this Operational status the integrity of the ATC service does not remains dependent on establishing and maintaining HF or VHF voice communications solely, as long as CPDLC communications are still available. Therefore, CPDLC is an additional support to ATC system integrity.

    - Operating with CPDLC does not release pilots from the obligation of establishing, maintaining, and monitoring HF or VHF voice communications.

# – RIGHTS AND RESPONSIBILITIES

## ATS Provider Rights and Responsibilities

3.1.1	The ATS provider may suspend temporarily the Operational status at any time due to technical or operational reasons.

### 3.1.2	The ATS provider will notify users of the current Phase of operations and version number of the Guidance Material applicable via NOTAM or AIC.

3.1.3	For scheduled and/or extended suspension of the operations a NOTAM shall be issued.

3.1.4	The ATS provider will issue NOTAMS for planned or predicted system outages.

3.1.5	In the event of an unexpected SACCAN V2 system  outage, ATS shall:

- inform all affected aircraft and their operators, providing the outage expected duration; and

- issue a NOTAM for long expected durations.

3.1.6	The ATS provider will provide operating airlines with latest version of Guidance Material, and will inform them of the publication of related AICS and NOTAMS. Latest version of  “GUIDANCE MATERIAL ON SACCAN V2 FANS 1/A OPERATIONAL STATUS IN CANARIAS AIRSPACE ” may be downloaded from the SATMA web site :

www.satmasat.com

3.2	Operator Rights and Responsibilities

### Operators, and pilots may suspend their participation in the Operational status anytime due to technical or operational reasons.

    - Operators should ensure they have current information related to the Operational status, such as Aeronautical Information Circulars, Guidance Material, and NOTAMS.

    - Operators should ensure that the proper information is included in the ICAO Flight Plan.

    - Advisory information distributed within the flight operations department of an airline should ensure that all personnel concerned are aware of FANS 1/A concepts and procedures and any other necessary information for proper system operations.

    - For scheduled and/or extended suspension of participation  in the Operational status operators should inform SATMA.

    - Pilots should inform CANARIAS ACC via voice communications if system outages are noted on route.

    - Operators should inform SATMA of any pilot reported problems or anomalies associated with the FANS 1/A Operational status.

    - To facilitate dissemination of outage information  all operators should provide SATMA with contacts for the receipt of outage or participation suspension information. Contact numbers should be sent to SATMA either via fax or e-mail.

3.3	Communications Service Provider Responsibilities

    - Data link service providers will inform SATMA and airline operators if a system outage is noted or of planned outages of the network.

# – OPERATIONAL PROCEDURES

4.1	Flight Planning Procedures

### To facilitate the eventual migration to a standardized CNS-ATM system, and in keeping with the flight planning provisions as specified in the ICAO Doc. 4444 (PANS-ATM), operators should complete the ICAO flight plan form as follows for FANS 1/A equipped aircraft:

- Item 10 – The letter “J” to indicate data link available and serviceable;

- Item 10 – The letter “G” to indicate GNSS available and serviceable;

- Item 10 – The letter “D” in the Surveillance field to indicate ADS serviceable;

- Item 18 –  DAT/  followed by one or more letters as appropriate to indicate the type of data link capability  when “J” is entered in Item 10.

- Item 18  –  RMK/  followed by CANARIAS FANS 1 or CANARIAS FANS A (specifically requested by  Canarias ACC to FANS 1/A operating aircraft)

Example:

ICAO Item 10: ….J….. / …D

ICAO Item 18: DAT/SV..RMK/CANARIAS FANS 1 (for a satellite and VHF data link equipped aircraft, and FANS 1 aircraft operating in the Canarias FANS 1/A Operational status)

| Letter following DAT/ | Type of data link |
| --- | --- |
| S | Satellite data link |
| H | HF data link |
| V | VHF data link |
| M | SSR mode S data link |

NOTE 1: The above requirements are for an end-state system. If an operator’s flight planning system does not have the capability to enter any of the data as indicated; this will not restrict participation in the SACCAN V2 FANS 1/A Operational Phase.

4.1.2	The operator is responsible for ensuring that the correct aircraft registration is filed in Field 18 of the ICAO flight plan. The ATS system (SACCAN V2) compares the registration number of the aircraft contained in Field 18 (Other Information) of the ICAO flight plan with the registration contained in the ATS Facilities Notification (AFN) logon in order to establish (if equal) or deny (if different) the connection with the aircraft.

## 4.2	Air Traffic Control Procedures

4.2.1		See “Log-on/Connection” procedures in 2.1.

    - See “Establishing and Terminating an ADS Connection” procedures in 2.2.

    - See “ADS Emergency Mode Operation” procedures in 2.3.2

    - See “Establishing, Executing and Terminating a CPDLC Connection” procedures in 2.5

    - Whenever an ADS-WPR is overdue by more than an interval, as determined by ATC, the controller will take action to advise the aircraft concerned in order to investigate or try to correct the situation.

    - A controller who becomes aware of corrupt or incorrect data, will take action to advise the aircraft concerned in order to investigate or try to correct the situation.

    - In order to perform the FANS1/A procedures the controller will report any anomalies or altered circumstances established to this respect.

    - If the controller is advised, or becomes aware of a data link communications failure, aircraft concerned will be advised.

    - When an unexpected/not programmed ADS emergency message is received, the controller with control responsibility for the aircraft shall request confirmation of the emergency, either through voice communications or CPDLC messages if available with the aircraft.

    - When a controller not having control responsibility for the aircraft receives an ADS emergency report, he/she shall co-ordinate with the controlling authority to ensure that the emergency report has been received and is investigated.

4.3	Flight Crew Procedures

4.3.1	When initialising the FMC, it is essential to ensure that the flight identification matches the one displayed in the filed ATC flight plan (FPL Message). If a flight crew becomes aware that they have provided incorrect flight identification data for logon to ATC, they shall immediately terminate ADS and re-logon  with a correct identification.

    - Pilots must be aware that appropriate clocks synchronisation to UTC (hours/minutes/seconds) is essential for FANS operations and specially for ADS. Although the time stamp source for ADS is GPS time referenced  to UTC, some aircraft use the manually set flight deck clock as a back up source when GPS time is unavailable.

    - Pilots might be asked by the controller to provide a “time check for ADS” by means of the following voice message: “REQUEST TIME CHECK FOR ADS”; if that is the case, pilot response should be : “TIME (minutes, seconds)”.

    - Pilots are requested to log-on between 15 and 30 minutes prior to entering the CANARIAS FIR/UIR. See “Log-on/Connection” procedures in 2.1

    - When entering the FANS ADS operational area from a non-FANS ADS sector, on initial voice contact with CANARIAS ACC the pilot will use the term “FANS ADS” after the call-sign.

    - If unable to log-on after several attempts , on initial voice contact with CANARIAS ACC the pilot should inform ATS using the following terminology:

“UNABLE TO FANS LOG-ON”

4.3.7	ADS, with the exception of the “ADS Emergency Mode Operation” will normally be initiated, managed, and terminated by the ground system, without pilot intervention.

    - See “ADS Emergency Mode Operation” procedures in 2.3.2

    - For CPDLC, see “Establishing, Executing and Terminating a CPDLC Connection” procedures in 2.5

    - If problems are experienced with the CPDLC connection , pilot should inform ATS via VHF or HF voice using the following terminology:

“PROBLEMS WITH CPDLC CONNECTION”

    - Log-off should normally take place 5 minutes after leaving CANARIAS FIR/UIR unless  something different is agreed in real time via pilot-controller voice communications.

    - Pilots of operating aircraft, 5 minutes after having left Canarias FIR/UIR, are requested either  to:

a) Log-off (Disconnect/Select off  ATC COMM ) the Canarias SACCAN V2 system

Or

b) Select off / Set off ADS

This is to make sure that ADS reporting is terminated in due time in case that due to a malfunction some ADS contracts may not be cancelled automatically by the SACCAN V2 system as they normally should be.

    - Flight crews that encounter problems with FANS 1/A data link will inform controller and advise their Company Operations Department in accordance with their established problem reporting procedures.

    - Operation with CPDLC does not release pilots from the obligation of establishing, maintaining, and monitoring HF or VHF voice communications.

    - In this Operational status the integrity of the ATC, that in past times remained wholly dependent on establishing and maintaining VHF or HF voice communications, will be supported with CPDLC communications as well. Therefore, an Aircraft could be unable of establishing positive two-way radio communications with Canarias ACC, but could establish two-way CPDLC communications. In these cases, ATC operations will remain, based on CPDLC as sole mean of transmitting ATC instructions and receiving aircraft communications.

APPENDIX 1

SACCAN V2 SYSTEM ARCHITECTURE AND BLOCKS DESCRIPTION

1.1	The major logical blocks of the SACCAN V2 system are as follows:

- Communications Processing (COM);

- Surveillance Data Processing (SDP);

- Flight Plan Data Processing (FDP);

- Aircraft Messages Processing (AMP);

- Radar Data Reception Unit (RRU);

- Supervision Terminal (SVT);

- GPS clock;

- Local Area Network (LAN);

- Tape unit;

- Color printer; and

- 1 Air Traffic Control Working Position made of :

-	1 Traffic Display Terminal (TDT); and

-	1 Data Link Terminal (DLT).

SITA

GIPV

TDT

DLT

GPSC

SDP

COM

TAPE

FDP

PRINT

SVT

AMP

LAN

RRU

COM : Communications Processing

SVT  :  Supervision Terminal

AMP :  Aircraft Messages Processing

SDP  :  Surveillance Data Processing

FDP  :  Flight Plan Data Processing

TDT  : Traffic Display Terminal

DLT :  Data Link Terminal

RRU :  Radar Data Reception Unit

GIPV:  Interface with SACTA FDP

GPSC:  GPS Clock

RADARS

FIGURE 2 : SACCAN V2 LOGICAL BLOCKS

1.2	Communications Processing (COM)

1.2.1	It deals with the external interfaces [SITA network, and GIPV (interface with the SACTA flight plan data processing system)], with the tape unit used for recording and playback, and with the GPS clock.

1.2.2	It shares a Sun Processor ULTRA 2 / 21" colour raster scan display (1280x1024 pixels) with the AMP and SVT logical blocks.

1.3	Surveillance Data Processing (SDP)

1.3.1	It deals with ADS and SSR data processing, tracks generation, STCA, and other surveillance related functions.

1.3.2	It shares a Sun Processor ULTRA 1 / 21" colour raster scan display (1280x1024 pixels) with the FDP logical block.

1.4	Flight Plan Data Processing (FDP)

1.4.1	It deals with all flight plan related functions such as creation, modification, and cancellation of flight plans. It takes pre-processed flight plans from the SACTA (current operational system) via the GIPV interface.

1.4.2	It shares a Sun Processor ULTRA 1 / 21" colour raster scan display (1280x1024 pixels) with the SDP logical block.

1.5	Aircraft Messages Processing (AMP)

1.5.1	It is the logical block where ADS, CPDLC and AFN (ATS Facilities Notification) messages are processed.

1.5.2	It shares a Sun Processor ULTRA 2 / 21" colour raster scan display (1280 x 1024 pixels) with the COM and SVT logical blocks.

### 1.6	Radar Data Reception Unit (RRU)

1.6.1	It pre-processes radar data from up to eight SSR radar stations. Currently only four are connected.

1.7	Supervision Terminal (SVT)

1.7.1	It deals with all supervision functions of the SACCAN V2 system.

1.7.2	It shares a Sun Processor ULTRA 2 / 21" colour raster scan display (1280 x 1024 pixels) with the COM and AMP logical blocks.

### 1.8	GPS Clock

1.8.1	It is a Trimble GPS receiver plus a time and frequency processor. This is mainly used to accurately time stamp incoming ADS messages so as to allow the Surveillance Data Processing (SDP) to measure the transit delay of ADS messages.

1.9	Local Area Network (LAN)

1.9.1	A LAN is used to interconnect the different subsystems of SACCAN V2.

1.10	Tape Unit

1.10.1 A tape unit is used for recording and play back.

1.11	Printer

1.11.1 A high-resolution colour printer Tektronix Phaser 560 is available. It is used to print any image being displayed at the Traffic Display Terminal, Data Link Terminal or Supervisor Terminal.

1.12	Traffic Display Terminal (TDT)

1.12.1 Consists of the same hardware as the new SACTA control position “FOCUCS”. This is so in order to facilitate the future integration of the SACCAN V2 new functions into the SACTA system. The Traffic Display Terminal consist in a colour Sony 2k x 2 k (2048x2048 pixels) 28" raster scan display, a Barco graphic processing card, and a Sun Processor Ultra 30.

1.12.2 Aircraft positions are displayed by means of appropriate symbols. Different symbols are used to differentiate between the different type of plots (PSR, SSR, SSR+PSR, ADS, ADS+SSR, ADS+SSR+PSR, extrapolations, and of synthetic tracks.).

1.12.3 The aircraft label, structured in four lines, in addition to the typical information (call-sign or SSR code, cleared flight level, current flight level, ground speed, vertical speed, etc.) contains:

- Type of alarm/alert, if any (EMG, COM FAILURE, HIJACK / STCA, MTCA, NIM);

- ADS on/off indicator;

- CPDLC on/off indicator;

- Predicted route on/off indicator;

- Conformance monitoring-flight plan update on/off indicator;

- Event indicator (Altitude range, vertical rate, lateral deviation, or  waypoint change);

- Data loss (positional periodic data) indication;

- Estimated maximum uncertainty (accuracy) of the displayed position (in metres) / Required accuracy (maximum uncertainty) of the displayed position (in metres);

- Time elapsed since last reception of positional data (in seconds) / Reporting rate of the ADS periodic contract in place (in seconds);

- Number of radars being used by the tracker (0 = only ADS); and

- Uncertainty of the displayed position (rounded to miles).

1.12.4 Also, on request of the controller, an uncertainty circle can be displayed around the aircraft position symbol, being its radius an indication of the correctness (uncertainty) of the aircraft displayed position [as in k)].

1.12.5 Most of the above information, which will be used mainly to assess the performance of the tracking algorithms during operations and evaluations, can be disable.

1.12.6 By pinching with the mouse the ADS on/off indicator, a window is opened for ADS contracts visualization and modification. By pinching the CPDLC on/off indicator, a window is opened to view the CPDLC messages exchange with the selected aircraft.

1.12.7 Tabulars do exist for:

- Short Term Conflict Alert (STCA) and Minimum Safe Altitude Warning (MSAW) based both on ADS and SSR data;

- Medium Term Conflict Alert (MTCA);

- Navigation Integrity Monitoring (NIM) Alert which is displayed when ADS and SSR positional data from the same aircraft do not match each other;

- Incorrect Waypoint Insertion Alert, which is displayed when “ADS next way-points” do not mach the flight plan;

- CPDLC messages display; and

- Flight Plans Display;

1.12.8 By pinching a specific call sign of the Flight Plans Tabular, an editing window is opened for flight plan visualization and modification.

1.12.9 From this terminal, also the airspace requirements mosaic can be visualized, and edited box by box if wanting to change the accuracy and redundancy requirements. Fast editing by areas is possible through the Supervision Terminal (SVT).

1.13	Data Link Terminal (DLT)

1.13.1 It is a Sun Processor ULTRA 1 with a 21" raster scan color display (1280x1024 pixels) which is mainly used for CPDLC messages exchange and for full manual management and visualization of ADS contracts. A mouse and a keyboard are available for messages input.

1.13.2 This terminal also shows the list of FANS 1/A equipped aircraft logged on to SACCAN V2, indicating if the ADS and CPDLC applications are active or not.

APPENDIX 2

Appendix 2:  Downlink and Uplink Messages

The following provides an overview of the downlink and uplink messages that can be used during this Operational phase of  SACCAN V2 FANS 1/A.

1.	Messages

A limited number of messages are available during current status of the Operational phase (see below). See Appendix 4 for a clear understanding of their meaning (intent).

1.1	Downlink Messages

It should be appreciated that whilst the airborne HMI will indicate that a message has been sent, there is no guarantee that it has been received at the SACCAN V2 controller position in CANARIAS ACC. Only one message at a time should be transmitted and free text use should be limited to a minimum.

Downlink messages are grouped in 2 categories, as follows:

Aircrew Requests

Aircrew Responses

1.2	Uplink Messages

Uplink messages are grouped in 8 categories, as follows:

Level Messages

Route Messages

Speed Messages

Time Messages

Voice Communication Instruction Messages

General Instruction Messages

Confirm Messages

Report Messages

2.	Downlink Messages

  - Requests

Aircrew Request

REQUEST CLIMB TO [altitude

REQUEST DESCENT TO [altitude

REQUEST [altitude

REQUEST DIRECT TO [position

REQUEST WEATHER DEVIATION UP TO [direction[distance offset OF ROUTE

REQUEST OFFSET [direction[distance offset OF ROUTE

REQUEST [route clearance

Note: Pilots should not expect controller voice read-backs to these pilot CPDLC requests, but a direct data link response. If the response to the request is not received within a reasonable time,  pilot should revert to R/T to repeat the request via voice.

See Examples 1 and 2 in Appendix 3

  - Responses

Of course, responses to ATC instructions, like WILCO, UNABLE, STANDBY, and ROGER, and reports responding to the ATC confirm/request messages described bellow (in 3.7) will be downlinked when appropriate.

3.	Uplink Messages

The following CPDLC messages can be expected from CANARIAS ACC:

Note: These controller instructions via CPDLC do not need to be confirmed via voice communications (R/T), as they have identical status as voice transmitted instructions, and therefore are to be complied with.

3.1	Level Messages

ATC Instruction

MAINTAIN [altitude

CLIMB TO AND MAINTAIN [altitude

DESCEND TO AND MAINTAIN [altitude

CROSS [position AT AND MAINTAIN [altitude

CROSS [position AT OR ABOVE [altitude

CROSS [position AT OR BELOW [altitude

See Example 3 in Appendix 3

3.2	Route Messages

ATC Instruction

PROCEED DIRECT TO [position

CLEARED [route clearance

OFFSET [direction[distance offset OF ROUTE

CLEARED TO DEVIATE UP TO [direction[distance offset OF ROUTE

- WHEN ABLE PROCEED DIRECT TO [position

TURN [direction HEADING [degrees

PROCEED BACK ON ROUTE

3.3	Speed Messages

ATC Instruction

MAINTAIN [speed (mach number)

MAINTAIN [speed OR GREATER (mach number)

MAINTAIN [speed OR LESS (mach number)

RESUME NORMAL SPEED

3.4	Time Messages

ATC Instruction

CROSS [position AT OR AFTER [time

CROSS [position AT OR BEFORE [time

3.5	Voice Communication Instruction (VCI) Messages

ATC Instruction

CONTACT [icao unit name [frequency AT [position] CONTACT [icao unit name [frequency MONITOR [icao unit name [frequency

Warning: MONITOR message will not be confirmed by voice readback. “MONITOR” message will be used with “SECONDARY HF” Free text (see below). CONTACT message will be complied by pilot and if change of frequency is in Canarias FIR/UIR, pilot will establish communication with GCCC on the new frequency.

3.6	General Instruction Messages

ATC Instruction

SQUAWK [beacon code

CHECK STUCK MICROPHONE [frequency

See Example 3 and Example 4 in Appendix 3

3.7	Confirm Messages

ATC Instruction

- CONFIRM NEXT WAYPOINT

CONFIRM NEXT WAYPOINT ETA

CONFIRM ASSIGNED ALTITUDE

- CONFIRM ASSIGNED ROUTE

Note: Controllers should not expect pilot voice read-backs to these controller CPDLC requests (confirms), but a direct data link response from aircraft. If the response to the request is not received within a reasonable time,  controller should revert to R/T to repeat the request via voice.

See Example 5 in Appendix 3

3.8	Report Messages

ATC Instruction

REPORT LEAVING [altitude

REPORT LEVEL [altitude

REPORT REACHING [altitude

REPORT BACK ON ROUTE

Note: Controllers should not expect pilot voice read-backs to these controller CPDLC messages.

See Example 6 in Appendix 3

3.9 	Use of Pre-Formatted and Free Text Messages

Free text messages shall be used only when an appropriate pre-formatted message element does not exist. In particular, the creation of a clearance request and the issuing of a clearance shall be performed by the use of pre-formatted message elements only. The use of pre-formatted message elements allows on board data processing such as the automatic insertion of the clearance information into the FMC. It also allows the controller to respond more quickly when the ATS system has the capability to automatically link a pre-formatted request to a pre-formatted response. Additionally, this process minimizes the risk of input errors.

When a free text message is required, standard ATC phraseology and format shall be used. Non-essential words and phrases should be avoided. Abbreviations should only be included in free text messages when they form part of standard ICAO phraseology, e.g. ETA.

3.9.1. Preferred use of pre-formatted messages. Standardized free text messages

While pre-formatted message elements are required to be used whenever possible, there are occasions where frequent use of free text allows the meaning and appropriate response to be standardized. The Standard Free text message set is shown below.

When a standard Free Text message is used, the pilot will respond with a QUICK RESPONSE before  answering the message.

### Uplink - Free Text Report/ Confirmation Requests (*)

|  | Free Text Message | Quick Response |
| --- | --- | --- |
| Controller | REPORT LEVEL REQUESTED |  |
| Pilot Response | FL [altitude] Example - FL 310 | ROGER |
| Message Intent | The controller is requesting that the pilot advise the preferred flight level for the flight. |  |

Note1: Controllers should not expect pilot voice read-backs to these controller CPDLC requests (confirms), but a direct data link response from aircraft. If the response to the request is not received within a reasonable time,  controller should revert to R/T to repeat the request via voice.

|  | Free Text Message | Quick Response |
| --- | --- | --- |
| Controller | REPORT EVEN LEVEL REQUESTED |  |
| Pilot Response | FL [altitude] Example - FL 360 | ROGER |
| Message Intent | The controller is requesting that the pilot advise the preferred even flight level for the flight. |  |

Note1: Controllers should not expect pilot voice read-backs to these controller CPDLC requests (confirms), but a direct data link response from aircraft. If the response to the request is not received within a reasonable time,  controller should revert to R/T to repeat the request via voice.

|  | Free Text Message | Quick Response |
| --- | --- | --- |
| Controller | REPORT ODD LEVEL REQUESTED |  |
| Pilot Response | FL [altitude] Example - FL 350 | ROGER |
| Message Intent | The controller is requesting that the pilot advise the preferred odd flight level for the flight. |  |

Note1: Controllers should not expect pilot voice read-backs to these controller CPDLC requests (confirms), but a direct data link response from aircraft. If the response to the request is not received within a reasonable time,  controller should revert to R/T to repeat the request via voice.

|  | Free Text Message | Quick Response |
| --- | --- | --- |
| Controller | REPORT ETA [position] Example – REPORT ETA EDUMO (Ref: ICAO UM228) |  |
| Pilot Response | [position] [time] Example - EDUMO 0213 | ROGER |
| Message Intent | The controller is requesting an estimate for the specified waypoint. |  |

Note: Controllers should not expect pilot voice read-backs to these controller CPDLC requests (confirms), but a direct data link response from aircraft. If the response to the request is not received within a reasonable time,  controller should revert to R/T to repeat the request via voice.

|  | Free Text Message | Response |
| --- | --- | --- |
| Pilot | REVISED ETA [position] [time] |  |
| Controller Response |  | ROGER |
| Message Intent | The pilot is advising ATC of an update a waypoint ETA. |  |

Note: Pilots should not expect controller voice read-backs to these pilot CPDLC messages, but a direct data link response. If the response is not received within a reasonable time,  pilot should revert to R/T to repeat via voice.

|  | Free Text Message | Quick Response |
| --- | --- | --- |
| Controller | SECONDARY HF [frequency] |  |
| Pilot Response |  | ROGER |
| Message Intent | The controller is notifying the pilot of the secondary HF frequency for the area. |  |

Warning: Response to this message does not require voice readback.

APPENDIX 3

Appendix 3:  Dialogues Examples

The following provides typical examples of pilot initiated and controller initiated dialogs that can occur during this Operational status of  SACCAN V2 FANS 1/A. The main purpose is to clarify the process of CPDLC controller instructions.

PILOT INITIATED DIALOGUES

Example 1

| Aircraft transmits | Ground transmits |
| --- | --- |
| Via Data Link | Via Data Link |
| REQUEST CLIMB TO 310 | CLIMB TO AND MAINTAIN 310 |
| WILCO {at this time, and not before, the aircraft executes the instruction} |  |

Example 2

| Aircraft transmits | Ground transmits |
| --- | --- |
| Via Data Link | Via Data Link |
| REQUEST DIRECT TO APASO | ROGER, PROCEED DIRECT TO APASO |
| WILCO {at this time, and not before, the aircraft executes the instruction} |  |

CONTROLLER INITIATED DIALOGUES

Example 3

| Aircraft transmits | Ground transmits |
| --- | --- |
| Via Data Link | Via Data Link |
|  | CLIMB TO AND MAINTAIN 310 |
| WILCO {at this time, and not before, the aircraft executes the instruction} |  |

Example 4

| Aircraft transmits | Ground transmits |
| --- | --- |
| Via Data Link | Via Data Link |
|  | IF YOU LOSE CONTACT IN 133.0, TRY CANARIAS HF 8861, SECONDARY 6535 |
| WILCO |  |

Example 5

| Aircraft transmits | Ground transmits |
| --- | --- |
| Via Data Link | Via Data Link |
|  | CONFIRM EDUMO ESTIMATE AND MACH NUMBER |
| EDUMO ESTIMATE 14:02, MACH NUMBER 080 |  |

Example 6

| Aircraft transmits | Ground transmits |
| --- | --- |
| Via Data Link | Via Data Link |
| ROGER | REPORT LEAVING 310 |
| Some minutes later: LEAVING 310 |  |

- Appendix 4 – FANS-1/A CPDLC Message Set and Intent

This Appendix contains a complete listing of the message intent for all FANS-1/A CPDLC messages as defined by the ADS Panel. Additional comments provided by the ISPACG forum are displayed in Italics.

Response Requirements Key:

| TYPE | CLOSURE RESPONSES |
| --- | --- |
| W/U | WILCO, UNABLE, will close the uplink message. |
| A/N | AFFIRM, NEGATIVE, will close the uplink message. |
| R | ROGER, will close the uplink message. |
| NE | Most messages with an NE attribute require an operational response. Only the correct operational response is presented to the pilot. The uplink message is considered to be closed on sending and does not require a response to close the dialogue. The WILCO, UNABLE, AFFIRM, NEGATIVE, ROGER, and STANDBY responses are not enabled for pilot selection. |
| Y | Response required. |
| N | Response not required |

Note: Under some circumstances, an ERROR message will also close an uplink message.

Uplink  - Responses and Acknowledgements

| UL | Message Element | Message Intent | Response |
| --- | --- | --- | --- |
| 0 | UNABLE | Indicates that ATS cannot comply with the request. | NE |
| 1 | STANDBY | Indicates that ATS has received the message and will respond. The pilot is informed that the request is being assessed and there will be a short-term delay (within 10 minutes). The exchange is not closed and the request will be responded to when conditions allow. | NE |
| 2 | REQUEST DEFERRED | Indicates that ATS has received the request but it has been deferred until later. The pilot is informed that the request is being assessed and a long-term delay can be expected. The exchange is not closed and the request will be responded to when conditions allow. | NE |
| 3 | ROGER | Indicates that ATS has received and understood the message.. | NE |
| 4 | AFFIRM | Yes | NE |
| 5 | NEGATIVE | No | NE |

Uplink - Vertical Clearances

| UL | Message Element | Message Intent | Response |
| --- | --- | --- | --- |
| 6 | EXPECT [altitude] | Notification that a level change instruction should be expected. | R |
| 7 | EXPECT CLIMB AT [time] | Notification that an instruction should be expected for the aircraft to commence climb at the specified time. | R |
| 8 | EXPECT CLIMB AT [position] | Notification that an instruction should be expected for the aircraft to commence climb at the specified position. | R |
| 9 | EXPECT DESCENT AT [time] | Notification that an instruction should be expected for the aircraft to commence descent at the specified time. | R |
| 10 | EXPECT DESCENT AT [position] | Notification that an instruction should be expected for the aircraft to commence descent at the specified position. | R |
| 11 | EXPECT CRUISE CLIMB AT [time] | Notification that an instruction should be expected for the aircraft to commence cruise climb at the specified time. Due to different interpretations between the various ATS units this element should be avoided. | R |
| 12 | EXPECT CRUISE CLIMB AT [position] | Notification that an instruction should be expected for the aircraft to commence cruise climb at the specified position. Due to different interpretations between the various ATS units this element should be avoided | R |
| 13 | AT [time] EXPECT CLIMB TO [altitude] | Notification that an instruction should be expected for the aircraft to commence climb at the specified time to the specified level. | R |
| 14 | AT [position] EXPECT CLIMB TO [altitude] | Notification that an instruction should be expected for the aircraft to commence climb at the specified position to the specified level. | R |
| 15 | AT [time] EXPECT DESCENT TO [altitude] | Notification that an instruction should be expected for the aircraft to commence descent at the specified time to the specified level. | R |
| 16 | AT [position] EXPECT DESCENT TO [altitude] | Notification that an instruction should be expected for the aircraft to commence descent at the specified position to the specified level. | R |
| 17 | AT [time] EXPECT CRUISE CLIMB TO [altitude] | Notification that an instruction should be expected for the aircraft to commence cruise climb at the specified time to the specified level. Due to different interpretations between the various ATS units, this element should be avoided. | R |

Uplink - Vertical Clearances Continued

| UL | Message Element | Message Intent | Response |
| --- | --- | --- | --- |
| 18 | AT [position] EXPECT CRUISE CLIMB TO [altitude] | Notification that an instruction should be expected for the aircraft to commence cruise climb at the specified position to the specified level. Due to different interpretations between the various ATS units, this element should be avoided. | R |
| 19 | MAINTAIN [altitude] | Instruction to maintain the specified level. | W/U |
| 20 | CLIMB TO AND MAINTAIN [altitude] | Instruction that a climb to the specified level is to commence and the level is to be maintained when reached. | W/U |
| 21 | AT [time] CLIMB TO AND MAINTAIN [altitude] | Instruction that at the specified time, a climb to the specified level is to commence and once reached the specified level is to be maintained. | W/U |
| 22 | AT [position] CLIMB TO AND MAINTAIN [altitude] | Instruction that at the specified position, a climb to the specified level is to commence and once reached the specified level is to be maintained. | W/U |
| 23 | DESCEND TO AND MAINTAIN [altitude] | Instruction that a descent to the specified level is to commence and the level is to be maintained when reached. | W/U |
| 24 | AT [time] DESCEND TO AND MAINTAIN [altitude] | Instruction that at the specified time a decent to the specified level is to commence and once reached the specified level is to be maintained. | W/U |
| 25 | AT [position] DESCEND TO AND MAINTAIN [altitude] | Instruction that at the specified position a descent to the specified level is to commence and when the specified level is reached it is to be maintained. | W/U |
| 26 | CLIMB TO REACH [altitude] BY [time] | Instruction that a climb is to commence at a rate such that the specified level is reached at or before the specified time. | W/U |
| 27 | CLIMB TO REACH [altitude] BY [position] | Instruction that a climb is to commence at a rate such that the specified level is reached at or before the specified position. | W/U |
| 28 | DESCEND TO REACH [altitude] BY [time] | Instruction that a descent is to commence at a rate such that the specified level is reached at or before the specified time. | W/U |
| 29 | DESCEND TO REACH [altitude] BY [position] | Instruction that a descent is to commence at a rate such that the specified level is reached at or before the specified position. | W/U |
| 30 | MAINTAIN BLOCK [altitude] TO [altitude] | A level within the specified vertical range is to be maintained. | W/U |
| 31 | CLIMB TO AND MAINTAIN BLOCK [altitude] TO [altitude] | Instruction that a climb to a level within the specified vertical range is to commence. | W/U |
| 32 | DESCEND TO AND MAINTAIN BLOCK [altitude] TO [altitude] | Instruction that a descent to a level within the specified vertical range is to commence. | W/U |

Uplink - Vertical Clearances Continued

| UL | Message Element | Message Intent | Response |
| --- | --- | --- | --- |
| 33 | CRUISE [altitude] | Instruction that authorizes a pilot to conduct flight at any altitude from the minimum altitude up to and including the altitude specified in the clearance. further, it is approval for the pilot to proceed to and make an approach at the destination airport. Due to different interpretations between the various ATS units, this element should be avoided. |  |
| 34 | CRUISE CLIMB TO [altitude] | A cruise climb is to commence and continue until the specified level is reached. Due to different interpretations between the various ATS units, this element should be avoided. | W/U |
| 35 | CRUISE CLIMB ABOVE [altitude] | A cruise climb can commence once above the specified level. Due to different interpretations between the various ATS units, this element should be avoided. | W/U |
| 36 | EXPEDITE CLIMB TO [altitude] | The climb to the specified level should be made at the aircraft's best rate. | W/U |
| 37 | EXPEDITE DESCENT TO [altitude] | The descent to the specified level should be made at the aircraft's best rate. | W/U |
| 38 | IMMEDIATELY CLIMB TO [altitude] | Urgent instruction to immediately climb to the specified level. | W/U |
| 39 | IMMEDIATELY DESCEND TO [altitude] | Urgent instruction to immediately descend to the specified level. | W/U |
| 40 | IMMEDIATELY STOP CLIMB AT [altitude] | Urgent instruction to immediately stop a climb once the specified level is reached. | W/U |
| 41 | IMMEDIATELY STOP DESCENT AT [altitude] | Urgent instruction to immediately stop a descent once the specified level is reached. | W/U |
| 171 | CLIMB AT [vertical rate] MINIMUM | Instruction to climb at not less than the specified rate. | W/U |
| 172 | CLIMB AT [vertical rate] MAXIMUM | Instruction to climb at not above the specified rate. | W/U |
| 173 | DESCEND AT [vertical rate] MINIMUM | Instruction to descend at not less than the specified rate. | W/U |
| 174 | DESCEND AT [vertical rate] MAXIMUM | Instruction to descend at not above the specified rate. | W/U |

Uplink - Crossing Constraints

| UL | Message Element | Message Intent | Response |
| --- | --- | --- | --- |
| 42 | EXPECT TO CROSS [position] AT [altitude] | Notification that a level change instruction should be expected which will require the specified position to be crossed at the specified level. | R |
| 43 | EXPECT TO CROSS [position] AT OR ABOVE [altitude] | Notification that a level change instruction should be expected which will require the specified position to be crossed at or above the specified level. | R |

Uplink - Crossing Constraints Continued

| UL | Message Element | Message Intent | Response |
| --- | --- | --- | --- |
| 44 | EXPECT TO CROSS [position] AT OR BELOW [altitude] | Notification that a level change instruction should be expected which will require the specified position to be crossed at or below the specified level. | R |
| 45 | EXPECT TO CROSS [position] AT AND MAINTAIN [altitude] | Notification that a level change instruction should be expected which will require the specified position to be crossed at the specified level which is to be maintained subsequently. | R |
| 46 | CROSS [position] AT [altitude] | The specified position is to be crossed at the specified level. This may require the aircraft to modify its climb or descent profile. | W/U |
| 47 | CROSS [position] AT OR ABOVE [altitude] | The specified position is to be crossed at or above the specified level. | W/U |
| 48 | CROSS [position] AT OR BELOW [altitude] | The specified position is to be crossed at or below the specified level. | W/U |
| 49 | CROSS [position] AT AND MAINTAIN [altitude] | Instruction that the specified position is to be crossed at the specified level and that level is to be maintained when reached. | W/U |
| 50 | CROSS [position] BETWEEN [altitude] AND [altitude] | The specified position is to be crossed at a level between the specified levels. | W/U |
| 51 | CROSS [position] AT [time] | The specified position is to be crossed at the specified time. | W/U |
| 52 | CROSS [position] AT OR BEFORE [time] | The specified position is to be crossed at or before the specified time. | W/U |
| 53 | CROSS [position] AT OR AFTER [time] | The specified position is to be crossed at or after the specified time. | W/U |
| 54 | CROSS [position] BETWEEN [time] AND [time] | The specified position is to be crossed at a time between the specified times. | W/U |
| 55 | CROSS [position] AT [speed] | The specified position is to be crossed at the specified speed and the specified speed is to be maintained until further advised. | W/U |
| 56 | CROSS [position] AT OR LESS THAN [speed] | The specified position is to be crossed at a speed equal to or less than the specified speed and the specified speed or less is to be maintained until further advised. | W/U |
| 57 | CROSS [position] AT OR GREATER THAN [speed] | The specified position is to be crossed at a speed equal to or greater than the specified speed and the specified speed or greater is to be maintained until further advised. | W/U |
| 58 | CROSS [position] AT [time] AT [altitude] | The specified position is to be crossed at the specified time and the specified level. | W/U |
| 59 | CROSS [position] AT OR BEFORE [time] AT [altitude] | The specified position is to be crossed at or before the specified time and at the specified level. | W/U |
| 60 | CROSS [position] AT OR AFTER [time] AT [altitude] | The specified position is to be crossed at or after the specified time and at the specified level. | W/U |
| 61 | CROSS [position] AT AND MAINTAIN [altitude] AT [speed] | Instruction that the specified position is to be crossed at the specified level and speed and the level and speed are to be maintained. | W/U |

Uplink - Crossing Constraints Continued

| UL | Message Element | Message Intent | Response |
| --- | --- | --- | --- |
| 62 | AT [time] CROSS [position] AT AND MAINTAIN [altitude] | Instruction that at the specified time the specified position is to be crossed at the specified level and the level is to be maintained. | W/U |
| 63 | AT [time] CROSS [position] AT AND MAINTAIN [altitude] AT [speed] | Instruction that at the specified time the specified position is to be crossed at the specified level and speed and the level and speed are to be maintained. | W/U |

Uplink - Lateral Offsets

| UL | Message Element | Message Intent | Response |
| --- | --- | --- | --- |
| 64 | OFFSET [direction] [distance offset] OF ROUTE | Instruction to fly a parallel track to the cleared route at a displacement of the specified distance in the specified direction. | W/U |
| 65 | AT [position] OFFSET [direction] [distance offset] OF ROUTE | Instruction to fly a parallel track to the cleared route at a displacement of the specified distance in the specified direction and commencing at the specified position. | W/U |
| 66 | AT [time] OFFSET [direction] [distance offset] OF ROUTE | Instruction to fly a parallel track to the cleared route at a displacement of the specified distance in the specified direction and commencing at the specified time. | W/U |
| 67 | PROCEED BACK ON ROUTE | The cleared flight route is to be rejoined. | W/U |
| 68 | REJOIN ROUTE BY [position] | The cleared flight route is to be rejoined at or before the specified position. | W/U |
| 69 | REJOIN ROUTE BY [time] | The cleared flight route is to be rejoined at or before the specified time. | W/U |
| 70 | EXPECT BACK ON ROUTE BY [position] | Notification that a clearance may be issued to enable the aircraft to rejoin the cleared route at or before the specified position. | R |
| 71 | EXPECT BACK ON ROUTE BY [time] | Notification that a clearance may be issued to enable the aircraft to rejoin the cleared route at or before the specified time. | R |
| 72 | RESUME OWN NAVIGATION | Instruction to resume own navigation following a period of tracking or heading clearances. May be used in conjunction with an instruction on how or where to rejoin the cleared route. | W/U |

Uplink - Route Modifications

| UL | Message Element | Message Intent | Response |
| --- | --- | --- | --- |
| 73 | [predepartureclearance] | Notification to the aircraft of the instructions to be followed from departure until the specified clearance limit. | W/U |
| 74 | PROCEED DIRECT TO [position] | Instruction to proceed directly from the present position to the specified position. | W/U |
| 75 | WHEN ABLE PROCEED DIRECT TO [position] | Instruction to proceed, when able, directly to the specified position. | W/U |

Uplink - Route Modifications Continued

| UL | Message Element | Message Intent | Response |
| --- | --- | --- | --- |
| 76 | AT [time] PROCEED DIRECT TO [position] | Instruction to proceed, at the specified time, directly to the specified position. | W/U |
| 77 | AT [position] PROCEED DIRECT TO [position] | lnstruction to proceed, at the specified position, directly to the next specified position. | W/U |
| 78 | AT [altitude] PROCEED DIRECT TO [position] | Instruction to proceed, upon reaching the specified level, directly to the specified position. | W/U |
| 79 | CLEARED TO [position] VIA [route clearance] | Instruction to proceed to the specified position via the specified route. | W/U |
| 80 | CLEARED [route clearance] | Instruction to proceed via the specified route. | W/U |
| 81 | CLEARED [procedure name] | Instruction to proceed in accordance with the specified procedure. | W/U |
| 82 | CLEARED TO DEVIATE UP TO [direction] [distance offset] OF ROUTE | Approval to deviate up to the specified distance from the cleared route in the specified direction. | W/U |
| 83 | AT [position] CLEARED [route clearance] | Instruction to proceed from the specified position via the specified route. | W/U |
| 84 | AT [position] CLEARED [procedure name] | Instruction to proceed from the specified position via the specified procedure. | W/U |
| 85 | EXPECT [route clearance] | Notification that a clearance to fly on the specified route may be issued. | R |
| 86 | AT [position] EXPECT [route clearance] | Notification that a clearance to fly on the specified route from the specified position may be issued. | R |
| 87 | EXPECT DIRECT TO [position] | Notification that a clearance to fly directly to the specified position may be issued. | R |
| 88 | AT [position] EXPECT DIRECT TO [position] | Notification that a clearance to fly directly from the first specified position to the next specified position may be issued. | R |
| 89 | AT [time] EXPECT DIRECT TO [position] | Notification that a clearance to fly directly to the specified position commencing at the specified time may be issued. | R |
| 90 | AT [altitude] EXPECT DIRECT TO [position] | Notification that a clearance to fly directly to the specified position commencing when the specified level is reached may be issued. | R |
| 91 | HOLD AT [position] MAINTAIN [altitude] INBOUND TRACK [degrees][direction] TURN LEG TIME [leg type] | Instruction to enter a holding pattern with the specified characteristics at the specified position and level. | W/U |
| 92 | HOLD AT [position] AS PUBLISHED MAINTAIN [altitude] | Instruction to enter a holding pattern with the published characteristics at the specified position and level. | W/U |
| 93 | EXPECT FURTHER CLEARANCE AT [time] | Notification that an onwards clearance may be issued at the specified time. | R |
| 94 | TURN [direction] HEADING [degrees] | Instruction to turn left or right as specified onto the specified heading. | W/U |
| 95 | TURN [direction] GROUND TRACK [degrees] | Instruction to turn left or right as specified onto the specified track. | W/U |
| 96 | FLY PRESENT HEADING | Instruction to continue to fly on the current heading. | W/U |

Uplink - Route Modifications Continued

| UL | Message Element | Message Intent | Response |
| --- | --- | --- | --- |
| 97 | AT [position] FLY HEADING [degrees] | Instruction to fly on the specified heading from the specified position. | W/U |
| 98 | IMMEDIATELY TURN [direction] HEADING [degrees] | Instruction to turn immediately left or right as specified onto the specified heading. | W/U |
| 99 | EXPECT [procedure name] | Notification that a clearance may be issued for the aircraft to fly the specified procedure. | R |
| 178 | TRACK DETAIL MESSAGE | Message not defined. |  |

Uplink - Speed Changes

| UL | Message Element | Message Intent | Response |
| --- | --- | --- | --- |
| 100 | AT [time] EXPECT [speed] | Notification that a speed instruction may be issued to be effective at the specified time. | R |
| 101 | AT [position] EXPECT [speed] | Notification that a speed instruction may be issued to be effective at the specified position. | R |
| 102 | AT [altitude] EXPECT [speed] | Notification that a speed instruction may be issued to be effective at the specified level. | R |
| 103 | AT [time] EXPECT [speed] TO [speed] | Notification that a speed range instruction may be issued to be effective at the specified time. | R |
| 104 | AT [position] EXPECT [speed] TO [speed] | Notification that a speed range instruction may be issued to be effective at the specified position. | R |
| 105 | AT [altitude] EXPECT [speed] TO [speed] | Notification that a speed range instruction may be issued to be effective at the specified level. | R |
| 106 | MAINTAIN [speed] | The specified speed is to be maintained. | W/U |
| 107 | MAINTAIN PRESENT SPEED | The present speed is to be maintained. | W/U |
| 108 | MAINTAIN [speed] OR GREATER | The specified speed or a greater speed is to be maintained. | W/U |
| 109 | MAINTAIN [speed] OR LESS | The specified speed or a lesser speed is to be maintained. | W/U |
| 110 | MAINTAIN [speed] TO [speed] | A speed within the specified range is to be maintained. | W/U |
| 111 | INCREASE SPEED TO [speed] | The present speed is to be increased to the specified speed and maintained until further advised. | W/U |
| 112 | INCREASE SPEED TO [speed] OR GREATER | The present speed is to be increased to the specified speed or greater, and maintained at or above the specified speed until further advised. | W/U |
| 113 | REDUCE SPEED TO [speed] | The present speed is to be reduced to the specified speed and maintained until further advised. | W/U |
| 114 | REDUCE SPEED TO [speed] OR LESS | The present speed is to be reduced to the specified speed or less and maintained at or below the specified speed until further advised. | W/U |

Uplink - Speed Changes Continued

| UL | Message Element | Message Intent | Response |
| --- | --- | --- | --- |
| 115 | DO NOT EXCEED [speed] | The specified speed is not to be exceeded. | W/U |
| 116 | RESUME NORMAL SPEED | Notification that the aircraft need no longer comply with the previously issued speed restriction. | W/U |

Uplink - Contact/Monitor/Surveillance Requests

| UL | Message Element | Message Intent | Response |
| --- | --- | --- | --- |
| 117 | CONTACT [icaounitname][frequency] | The pilot is required to call the ATS facility on the specified frequency. | W/U |
| 118 | AT [position] CONTACT [icaounitname] [frequency] | At the specified position the ATS unit with the specified ATS unit name is to be contacted on the specified frequency. | W/U |
| 119 | AT [time] CONTACT [icaounitname] [frequency] | At the specified time the ATS unit with the specified ATS unit name is to be contacted on the specified frequency. | W/U |
| 120 | MONITOR [icaounitname][frequency] | The pilot is required to monitor the specified ATS facility on the specified frequency. The Pilot is not required to check in. | W/U |
| 121 | AT [position] MONITOR [icaounitname] [frequency] | At the specified position the ATS unit with the specified ATS unit name is to be monitored on the specified frequency. | W/U |
| 122 | AT [time] MONITOR [icaounitname] [frequency] | At the specified time the ATS unit with the specified ATS unit name is to be monitored on the specified frequency. | W/U |
| 123 | SQUAWK [beacon code] | The specified code (SSR code) is to be selected. | W/U |
| 124 | STOP SQUAWK | The SSR transponder responses are to be disabled. | W/U |
| 125 | SQUAWK ALTITUDE | The SSR transponder responses should include level information. | W/U |
| 126 | STOP ALTITUDE SQUAWK | The SSR transponder responses should no longer include level information. | W/U |
| 179 | SQUAWK IDENT | The 'ident' function on the SSR transponder is to be actuated. | W/U |

Uplink - Report/Confirmation Requests

| UL | Message Element | Message Intent | Response |
| --- | --- | --- | --- |
| 127 | REPORT BACK ON ROUTE | Instruction to report when the aircraft is back on the cleared route. | R |
| 128 | REPORT LEAVING [altitude] | Instruction to report when the aircraft has left the specified level. Either a level that has been maintained, or a level passed through on climb or descent. | R |
| 129 | REPORT LEVEL [altitude] | Instruction to report when the aircraft is in level flight at the specified level. Some States do not to use this message in order to avoid confusion because it does not comply with existing voice phraseology | R |

Uplink - Report/Confirmation Requests Continued

| UL | Message Element | Message Intent | Response |
| --- | --- | --- | --- |
| 175 | REPORT REACHING [altitude] | Instruction to report when the aircraft has reached the specified level. To be interpreted as “Report reaching an assigned level.” | R |
| 180 | REPORT REACHING BLOCK [altitude] TO [altitude] | Instruction to report when the aircraft is within the specified vertical range. | R |
| 130 | REPORT PASSING [position] | Instruction to report when the aircraft has passed the specified position. | R |
| 181 | REPORT DISTANCE [to/from] [position] | Instruction to report the present distance to or from the specified position. | NE |
| 131 | REPORT REMAINING FUEL AND SOULS ON BOARD | Instruction to report the amount of fuel remaining and the number of persons on board. | NE |
| 132 | CONFIRM POSITION | Instruction to report the present position. | NE |
| 133 | CONFIRM ALTITUDE | Instruction to report the present level. | NE |
| 134 | CONFIRM SPEED | Instruction to report the present speed. | NE |
| 135 | CONFIRM ASSIGNED ALTITUDE | Instruction to confirm and acknowledge the currently assigned level. | NE |
| 136 | CONFIRM ASSIGNED SPEED | Instruction to confirm and acknowledge the currently assigned speed. | NE |
| 137 | CONFIRM ASSIGNED ROUTE | Instruction to confirm and acknowledge the currently assigned route. | NE |
| 138 | CONFIRM TIME OVER REPORTED WAYPOINT | Instruction to confirm the previously reported time over the last reported waypoint. | NE |
| 139 | CONFIRM REPORTED WAYPOINT | Instruction to confirm the identity of the previously reported waypoint. | NE |
| 140 | CONFIRM NEXT WAYPOINT | lnstruction to confirm the identity of the next waypoint. | NE |
| 141 | CONFIRM NEXT WAYPOINT ETA | Instruction to confirm the previously reported estimated time at the next waypoint. | NE |
| 142 | CONFIRM ENSUING WAYPOINT | Instruction to confirm the identity of the next plus one waypoint. | NE |
| 143 | CONFIRM REQUEST | The request was not understood. It should be clarified and resubmitted. | NE |
| 144 | CONFIRM SQUAWK | Instruction to report the currently selected transponder code. | NE |
| 145 | CONFIRM HEADING | Instruction to report the present heading. | NE |
| 146 | CONFIRM GROUND TRACK | Instruction to report the present ground track. | NE |
| 182 | CONFIRM ATIS CODE | Instruction to report the identification code of the last ATIS received. | NE |
| 147 | REQUEST POSITION REPORT | Instruction to make a position report. To be used if the controller does not receive a scheduled position report. | NE |

Uplink - Negotiation Requests

| UL | Message Element | Message Intent | Response |
| --- | --- | --- | --- |
| 148 | WHEN CAN YOU ACCEPT [altitude] | Request for the earliest time at which the specified level can be accepted. | NE |
| 149 | CAN YOU ACCEPT [altitude] AT [position] | Instruction to report whether or not the specified level can be accepted at the specified position. | A/N |
| 150 | CAN YOU ACCEPT [altitude] AT [time] | Instruction to report whether or not the specified level can be accepted at the specified time. | A/N |
| 151 | WHEN CAN YOU ACCEPT [speed] | Instruction to report the earliest time when the specified speed can be accepted. | NE |
| 152 | WHEN CAN YOU ACCEPT [direction] [distance offset] OFFSET | Instruction to report the earliest time when the specified offset track can be accepted. | NE |

Uplink - Air Traffic Advisories

| UL | Message Element | Message Intent | Response |
| --- | --- | --- | --- |
| 153 | ALTIMETER [altimeter] | ATS advisory that the altimeter setting should be the specified setting. | R |
| 154 | RADAR SERVICES TERMINATED | ATS advisory that the radar service is terminated. | R |
| 155 | RADAR CONTACT [position] | ATS advisory that radar contact has been established at the specified position. | R |
| 156 | RADAR CONTACT LOST | ATS advisory that radar contact has been lost. | R |
| 157 | CHECK STUCK MICROPHONE [frequency] | A continuous transmission is detected on the specified frequency. Check the microphone button. | R |
| 158 | ATIS [atis code] | ATS advisory that the ATIS information identified by the specified code is the current ATIS information. | R |

Uplink - System Management Messages

| UL | Message Element | Message Intent | Response |
| --- | --- | --- | --- |
| 159 | ERROR [error information] | A system generated message that the ground system has detected an error. | NE |
| 160 | NEXT DATA AUTHORITY [facility designation] | Notification to the avionics that the next data authority is the specified ATSU. | NE |
| 161 | END SERVICE | Notification to the avionics that the data link connection with the current data authority is being terminated. | NE |
| 162 | SERVICE UNAVAILABLE | Notification that the ground system does not support this message. | NE |
| 163 | [icao facility designation] [tp4Table] | Notification to the pilot of an ATSU identifier. | NE |

Uplink - Additional Messages

| UL | Message Element | Message Intent | Response |
| --- | --- | --- | --- |
| 164 | WHEN READY | The associated instruction may be complied with at any future time. | NE |
| 165 | THEN | Used to link two messages, indicating the proper order of execution of clearances/ instructions. | NE |
| 166 | DUE TO TRAFFIC | The associated instruction is issued due to traffic considerations. | NE |
| 167 | DUE TO AIRSPACE RESTRICTION | The associated instruction is issued due to airspace restrictions. | NE |
| 168 | DISREGARD | The indicated communication should be ignored. The previously sent uplink CPDLC message shall be ignored. DISREGARD should not refer to a clearance or instruction. If DISREGARD is used, another element shall be added to clarify which message is to be disregarded. | R |
| 176 | MAINTAIN OWN SEPARATION AND VMC | Notification that the pilot is responsible for maintaining separation from other traffic and is also responsible for maintaining Visual Meteorological Conditions. | W/U |
| 177 | AT PILOTS DISCRETION | Used in conjunction with a clearance or instruction to indicate that the pilot may execute when prepared to do so. | N |
| 169 | [free text] | Normal urgency attribute | R |
| 170 | [free text] | Distress urgency attribute | R |

Downlink - Responses

| DL | Message Element | Message Intent | Response |
| --- | --- | --- | --- |
| 0 | WILCO | The instruction is understood and will be complied with. | N |
| 1 | UNABLE | The instruction cannot be complied with. | N |
| 2 | STANDBY | Wait for a reply. The controller is informed that the request is being assessed and there will be a short term delay (within 10 minutes). The exchange is not closed and the request will be responded to when conditions allow. | N |
| 3 | ROGER | Message received and understood. ROGER is the only correct response to an uplink free text message. Under no circumstances will ROGER be used instead of AFFIRM. | N |
| 4 | AFFIRM | Yes Affirm is an appropriate response to an uplinked negotiation request message (e.g. Can you accept [altitude] at [time]). | N |
| 5 | NEGATIVE | No Negative is an appropriate response to an uplinked negotiation request message (e.g. Can you accept [altitude] at [time]). | N |

Downlink - Vertical Requests

| DL | Message Element | Message Intent | Response |
| --- | --- | --- | --- |
| 6 | REQUEST [altitude] | Request to fly at the specified level. | Y |
| 7 | REQUEST BLOCK [altitude] TO [altitude] | Request to fly at a level within the specified vertical range. | Y |
| 8 | REQUEST CRUISE CLIMB TO [altitude] | Request to cruise climb to the specified level. Due to different interpretations between the various ATS units, this element should be avoided. | Y |
| 9 | REQUEST CLIMB TO [altitude] | Request to climb to the specified level. | Y |
| 10 | REQUEST DESCENT TO [altitude] | Request to descend to the specified level. | Y |
| 11 | AT [position] REQUEST CLIMB TO [altitude] | Request that at the specified position a climb to the specified level be approved. | Y |
| 12 | AT [position] REQUEST DESCENT TO [altitude] | Request that at the specified position a descent to the specified level be approved. | Y |
| 13 | AT [time] REQUEST CLIMB TO [altitude] | Request that at the specified time a climb to the specified level be approved. | Y |
| 14 | AT [time] REQUEST DESCENT TO [altitude] | Request that at the specified time a descent to the specified level be approved. | Y |

Downlink - Lateral Off-Set Requests

| DL | Message Element | Message Intent | Response |
| --- | --- | --- | --- |
| 15 | REQUEST OFFSET [direction] [distance offset] OF ROUTE | Request that a parallel track, offset from the cleared track by the specified distance in the specified direction, be approved. | Y |
| 16 | AT [position] REQUEST OFFSET [direction] [distance offset] OF ROUTE | Request that a parallel track, offset from the cleared track by the specified distance in the specified direction, be approved from the specified position. | Y |
| 17 | AT [time] REQUEST OFFSET [direction] [distance offset] OF ROUTE | Request that a parallel track, offset from the cleared track by the specified distance in the specified direction, be approved from the specified time. | Y |

Downlink - Speed Requests

| DL | Message Element | Message Intent | Response |
| --- | --- | --- | --- |
| 18 | REQUEST [speed] | Request to fly at the specified speed. | Y |
| 19 | REQUEST [speed] TO [speed] | Request to fly within the specified speed range. | Y |

Downlink - Voice Contact Requests

| DL | Message Element | Message Intent | Response |
| --- | --- | --- | --- |
| 20 | REQUEST VOICE CONTACT | Request for voice contact. | Y |
| 21 | REQUEST VOICE CONTACT [frequency] | Request for voice contact on the specified frequency. | Y |

Downlink - Route Modification Requests

| DL | Message Element | Message Intent | Response |
| --- | --- | --- | --- |
| 22 | REQUEST DIRECT TO [position] | Request to track from the present position direct to the specified position. | Y |
| 23 | REQUEST [procedure name] | Request for the specified procedure clearance. | Y |
| 24 | REQUEST [route clearance] | Request for a route clearance. | Y |
| 25 | REQUEST CLEARANCE | Request for either a pre-departure or route clearance. | Y |
| 26 | REQUEST WEATHER DEVIATION TO [position] VIA [route clearance] | Request for a weather deviation to the specified position via the specified route. | Y |
| 27 | REQUEST WEATHER DEVIATION UP TO [direction] [distance offset] OF ROUTE | Request for a weather deviation up to the specified distance off track in the specified direction. | Y |
| 70 | REQUEST HEADING [degrees] | Request a clearance to adopt the specified heading. | Y |
| 71 | REQUEST GROUND TRACK [degrees] | Request a clearance to adopt the specified ground track. | Y |

Downlink - Reports

| DL | Message Element | Message Intent | Response |
| --- | --- | --- | --- |
| 28 | LEAVING [altitude] | Notification of leaving the specified level. | N |
| 29 | CLIMBING TO [altitude] | Notification of climbing to the specified level. | N |
| 30 | DESCENDING TO [altitude] | Notification of descending to the specified level. | N |
| 31 | PASSING [position] | Notification of passing the specified position. | N |
| 78 | AT [time] [distance] [to/from] [position] | At the specified time, the aircraft's position was as specified. | N |
| 32 | PRESENT ALTITUDE [altitude] | Notification of the present level. | N |
| 33 | PRESENT POSITION [position] | Notification of the present position. | N |
| 34 | PRESENT SPEED [speed] | Notification of the present speed. | N |
| 35 | PRESENT HEADING [degrees] | Notification of the present heading in degrees. | N |
| 36 | PRESENT GROUND TRACK [degrees] | Notification of the present ground track in degrees. | N |
| 37 | LEVEL [altitude] | Notification that the aircraft is maintaining the specified level. | N |
| 72 | REACHING [altitude] | Notification that the aircraft has reached the specified level. | N |
| 76 | REACHING BLOCK [altitude] TO [altitude] | Notification that the aircraft has reached a level within the specified vertical range. | N |
| 38 | ASSIGNED ALTITUDE [altitude] | Read-back of the assigned level. | N |
| 77 | ASSIGNED BLOCK [altitude] TO [altitude] | Read-back of the assigned vertical range. | N |
| 39 | ASSIGNED SPEED [speed] | Read-back of the assigned speed. | N |
| 40 | ASSIGNED ROUTE [route clearance] | Read-back of the assigned route. | N |
| 41 | BACK ON ROUTE | The aircraft has regained the cleared route. | N |
| 42 | NEXT WAYPOINT [position] | The next waypoint is the specified position. | N |
| 43 | NEXT WAYPOINT ETA [time] | The ETA at the next waypoint is as specified. | N |
| 44 | ENSUING WAYPOINT [position] | The next plus one waypoint is the specified position. | N |
| 45 | REPORTED WAYPOINT [position] | Clarification of previously reported waypoint passage. | N |
| 46 | REPORTED WAYPOINT [time] | Clarification of time over previously reported waypoint. | N |
| 47 | SQUAWKING [beacon code] | The specified (SSR) code has been selected. | N |
| 48 | POSITION REPORT [position report] | Reports the current position of the aircraft when the pilot presses the button to send this message. ATC expects position reports based on this downlink message | N |

Downlink - Reports Continued

| DL | Message Element | Message Intent | Response |
| --- | --- | --- | --- |
| 79 | ATIS [atis code] | The code of the latest ATIS received is as specified. | N |
| 80 | DEVIATING [direction] [distance offset] OF ROUTE | Notification that the aircraft is deviating from the cleared route by the specified distance in the specified direction. | N |

Downlink - Negotiation Requests

| DL | Message Element | Message Intent | Response |
| --- | --- | --- | --- |
| 49 | WHEN CAN WE EXPECT [speed] | Request for the earliest time at which a clearance to the specified speed can be expected. | Y |
| 50 | WHEN CAN WE EXPECT [speed] TO [speed] | Request for the earliest time at which a clearance to a speed within the specified range can be expected. | Y |
| 51 | WHEN CAN WE EXPECT BACK ON ROUTE | Request for the earliest time at which a clearance to regain the planned route can be expected. | Y |
| 52 | WHEN CAN WE EXPECT LOWER ALTITUDE | Request for the earliest time at which a clearance to descend can be expected. | Y |
| 53 | WHEN CAN WE EXPECT HIGHER ALTITUDE | Request for the earliest time at which a clearance to climb can be expected. | Y |
| 54 | WHEN CAN WE EXPECT CRUISE CLIMB TO [altitude] | Request for the earliest time at which a clearance to cruise climb to the specified level can be expected. | Y |

Downlink - Emergency Messages

| DL | Message Element | Message Intent | Response |
| --- | --- | --- | --- |
| 55 | PAN PAN PAN | Urgency prefix. | N |
| 56 | MAYDAY MAYDAY MAYDAY | Distress prefix. | N |
| 57 | [remaining fuel] OF FUEL REMAINING AND [souls on board] SOULS ON BOARD | Notification of fuel remaining and number of persons on board. | N |
| 58 | CANCEL EMERGENCY | Notification that the pilot wishes to cancel the emergency condition. | N |
| 59 | DIVERTING TO [position] or DIVERTING TO [position] VIA [x] | Notification that the aircraft is diverting to the specified position via the specified route. | N |
| 60 | OFFSETTING [direction] [distance offset] OF ROUTE | Notification that the aircraft is deviating the specified distance in the specified direction off the cleared route and maintaining a parallel track. | N |
| 61 | DESCENDING TO [altitude] | Notification that the aircraft is descending to the specified level. | N |

Downlink -System Management Messages

| DL | Message Element | Message Intent | Response |
| --- | --- | --- | --- |
| 62 | ERROR [error information] | A system generated message that the avionics has detected an error. | N |
| 63 | NOT CURRENT DATA AUTHORITY | A system generated denial to any CPDLC message sent from a ground facility that is not the Current Data Authority. | N |
| 64 | [icao facility designation] | Notification to the ground system that the specified ATSU is the current data authority. | N |
| 73 | [version number] | A system generated message indicating the software version number. | N |

Downlink -Additional Messages

| DL | Message Element | Message Intent | Response |
| --- | --- | --- | --- |
| 65 | DUE TO WEATHER | Used to explain reasons for aircraft operator’s message. | N |
| 66 | DUE TO AIRCRAFT PERFORMANCE | Used to explain reasons for aircraft operator's message. | N |
| 74 | MAINTAIN OWN SEPARATION AND VMC | States a desire by the pilot to provide his/her own separation and remain in VMC. | N |
| 75 | AT PILOTS DISCRETION | Used in conjunction with another message to indicate that the pilot wishes to execute the request when the pilot is prepared to do so. | N |
| 67 | [free text] | Normal urgency attribute | N |
| 67b | WE CAN ACCEPT [altitude] AT [time] | We can accept the specified level at the specified time. | N |
| 67c | WE CAN ACCEPT [speed] AT [time] | We can accept the specified speed at the specified time. | N |
| 67d | WE CAN ACCEPT [direction] [distance offset] AT [time] | We can accept a parallel track offset the specified distance in the specified direction at the specified time. | N |
| 67e | WE CANNOT ACCEPT [altitude] | We cannot accept the specified level. | N |
| 67f | WE CANNOT ACCEPT [speed] | We cannot accept the specified speed. | N |
| 67g | WE CANNOT ACCEPT [direction] [distance offset] | We cannot accept a parallel track offset the specified distance in the specified direction. | N |
| 67h | WHEN CAN WE EXPECT CLIMB TO [altitude] | Request for the earliest time at which a clearance to climb to the specified level can be expected. | N |
| 67i | WHEN CAN WE EXPECT DESCENT TO [altitude] | Request for the earliest time at which a clearance to descend to the specified level can be expected. | N |
| 68 | [free text] | Distress urgency attribute | Y |

Intentionally Blank

APPENDIX 5

SACCAN V2 FANS 1/A OPERATIONAL PHASE

REGISTRATION FORM

Operator Name:________________________________________

Operator contact person

Name:________________________________________________________________

Post/Position:_________________________________

Postal address:________________________________________________________

Telephone number:____________________     Fax number:_______________________

Email address:________________________________

Aircraft information (include all participating aircraft)

Aircraft type	     Registration	       GPS time*   Option of participation**      Start date

__________          ______________         _____              ____________             ________

__________          ______________         _____              ____________             ________

__________          ______________         _____              ____________             ________

__________          ______________         _____              ____________             ________

__________          ______________         _____              ____________             ________

__________          ______________         _____              ____________             ________

__________          ______________         _____              ____________             ________

__________          ______________         _____              ____________             ________

__________          ______________         _____              ____________             ________

.                                  .                             .                                 .                                   .

.                                  .                             .                                 .                                   .

(extend as required)

*whether the option of updating the FMC time using the GPS time has been installed (YES/NO)

**whether the option of participation is for ADS only, or also for CPDLC subject to CPDLC

trained crew on board ( ADS / ADS&CPDLC )

Type of  approval (indicate whether is limited to ADS or valid for both ADS&CPDLC )

Remarks / Additional Information